To make an updated version of the DOS 'shell', here's

what to do:

1) Make appropriate corrections to the applicable .src

file.

2) Assemble the source. The file to assemble is shell.src,

which is just a bunch of .include files. To assemble, type:

run sys$cad:[cssyssoft]cy6502

This will run the 6502 assembler. At the assembler prompt,

which, for the moment, escapes me, respond

shell,shell=shell

The assembler will tell you about any errors or warnings

resulting from assembling your new code.

3) If no errors occoured, type ctl-z to exit the assembler.

There now exists an object (.obj) file. This file now needs

to be converted into Commodore (MOS) format. The program

to do this if invoked by typing:

run sys$cad:[cssyssoft]objcnv

After getting yet another curious prompt, respond:

shell/f:mtk=shell/f:bso

or, in english, convert the bso format object code to MOS

(they got us confused with mostek) format. It is very

unlikely that any errors will be generated at this stage.

Once again, exit the program with a ctl-z.

4) Now the shell object code exists on the VAX, and it

will be necessary to download it to a floppy disk. The

most straightforward way of doing this isn't. The machine

best suited to rs-232 communication is the plus/4. The

6551 ACIA in this machine is much better suited to prolonged

high speed communication than any of the software acia's

in other machines.

Get a plus/4. Get a 1011A cartridge, and remove it from it's

case (so it will fit in a Plus/4). Connect a cable from the

1011A (with pins 4&5 tied together, and 6&8&20 tied to each

other) to the printer port of your C.Itoh terminal.

Go into the setup mode of your terminal, and select screen D.

Set the printer port baud rate to 9600 (tx & rx, of course).

Find a copy of the Plus/4 download program RSXxxx (whatever

the version of the day happens to be). Load and run the

program on the Plus/4. At the offset prompt, enter 2000.

The implication here is that the shell program, which

normally resides at $1800, will be loaded into ram starting

at $3800. No problem, we'll fix this later. Now run the

program DOWNLOAD.COM on this tape.

DOWNLOAD shell.obj

If everything is running at the same baud rate, and if all

your cables are wired correctly, and if ...., the screen

on the Plus/4 should go blank for a few minutes, only

to return with a list of segment addresses. The ending

address of the last segment is the one to be concerned with.

'Pop' into the monitor on the Plus/4,

MONITOR

and save the file (in the following example, substitute

'xxxx' with the last end-of-segment mentioned above):

S"SHELL.3800.xxxx",8,3800,xxxx

Assuming you had a disk in drive 8, and didn't have a

write protect on, the save should occour without any

disk errors.

5) Now move over to a C-128. Put a copy of the OLD

test/demo diskette into the drive, and delete the current

version of the dos shell. (Don't forget to remove the write

protect!).

SCRATCH "DOS SHELL"

Now pop into the monitor,

MONITOR

and put the disk with SHELL.3800.xxxx in the drive. Load

it into memory,

L"SHELL.3800.xxxx",8

Now it will be necessary to relocate it to its proper address,

$1800.

T 3800 xxxx 1800

If you just want to run the program at this point, you need

only G to it. Remember, however, that it assumes bank 12,

and the actual starting address is $1a00.

G C1A00

If you want to put this program on the disk, do:

S "DOS SHELL",8,1800,@@@@

where '@@@@' is 'xxxx'-$2000. (If you are among the faint

of heart, you might want to round 'xxxx' and '@@@@' up

to, say, the nearest paragraph boundry).

The reason this was put on an existing test/demo disk is

to avoid the hassles of making the disk a boot disk. This

is normally an easy task, thanks to the efforts of

Mr. Bowen and his magic program, but the author of the

dos shell has complicated the task by making use of the

boot routine's imbedded-binary carry along. If it should

become necessary to modify the boot sector itself, contact

a programmer.