C-64/128 KERMIT USER'S GUIDE                          PAGE 3

INTRODUCTORY TUTORIAL

BOOTING KERMIT

KERMIT IS WRITTEN FOR THE COMMODORE 64 AND 128 (IN 64 MODE).

ONE COMMODORE DISK DRIVE (OR COMPATIBLE) IS REQUIRED, AND A

MODEM.  ALMOST ANY MODEM WILL WORK EXCEPT THE COMMODORE

1660.  SUPPORT FOR THE 1660 MODEM WILL BE INCLUDED IN A

FUTURE VERSION.  TO BOOT KERMIT, FOLLOW THESE EASY STEPS:

FOR COMMODORE 64:

1. TURN ON MONITOR, DRIVE, AND COMPUTER.

2. INSERT THE KERMIT DISK, LABEL SIDE UP, INTO DEVICE 8,

DRIVE 0, AND CLOSE THE DRIVE DOOR.

3. TYPE: LOAD "BOOT",8,1 AND PRESS <RETURN>.

FOR COMMODORE 128:

1. INSERT THE KERMIT DISK, LABEL SIDE UP, INTO DEVICE 8,

DRIVE 0, AND CLOSE THE DRIVE DOOR.

2. TURN ON THE MONITOR, DRIVE, AND COMPUTER.

3. KERMIT WILL AUTOMATICALLY BOOT ON POWER-UP.

ONCE LOADED, YOU SHOULD SEE THE FOLLOWING IN BLACK ON A GREY

SCREEN WITH A BLUE BORDER:

COMMODORE 64/128 KERMIT VERSION 2.1 (68)

TYPE '?' FOR HELP

KERMIT-65>

A BLINKING BLOCK CURSOR SHOULD BE JUST TO THE RIGHT OF THE

">".  IF YOU DON'T SEE THIS ON YOUR SCREEN, TURN OFF ALL

YOUR EQUIPMENT AND TRY AGAIN.  IF YOU STILL HAVE PROBLEMS

BOOTING KERMIT, PLEASE SEE THE TROUBLESHOOTING SECTION.

CHOOSING A SCREEN DRIVER

KERMIT CAN USE MANY KINDS OF DISPLAYS.  YOU CAN USE EITHER

40 COLUMNS (NOT VERY USEFUL) OR 80 COLUMNS (SOFTWARE-DRIVEN)

ON A STANDARD C-64, OR, IF YOU HAVE A BATTERIES INCLUDED

80-COLUMN CARD, YOU CAN USE IT TOO.  ALL OF THESE OPTIONS

ARE AVAILABLE ON A C-128, PLUS THE USE OF THE 128'S BUILT-IN

80-COLUMN DISPLAY.  KERMIT DEFAULTS TO THE C-64 80-COLUMN

SCREEN DRIVER.

EACH OF THE SCREEN DRIVERS HAS ITS ADVANTAGES AND DISADVAN-

TAGES.  NOT ALL OF THE DRIVERS, FOR EXAMPLE, ARE CAPABLE OF

SUPPORTING FEATURES SUCH AS UNDERLINING OR BOLD.  PLEASE SEE

PAGE 4                          C-64/128 KERMIT USER'S GUIDE

THE SCREEN DRIVER COMPARISON SECTION FOR FURTHER DETAILS.

IF YOU WANT TO SWITCH TO A DIFFERENT SCREEN DRIVER NOW,

PLEASE SEE THE EXAMPLE GIVEN IN THE COMMAND MODE SECTION

BELOW.

THE THREE MODES OF KERMIT

KERMIT HAS THREE BASIC OPERATIONAL MODES: COMMAND, CONNECT,

AND FILE-TRANSFER.  IN COMMAND MODE, YOU CAN COMMUNICATE

DIRECTLY WITH KERMIT TO CHANGE VARIOUS PROGRAM SETTINGS AND

ALSO ENTER EITHER OF THE TWO OTHER MODES.  IN CONNECT MODE

YOU CAN COMMUNICATE WITH ANOTHER COMPUTER (KNOWN IN MANY

OTHER TERMINAL PROGRAMS AS TERMINAL MODE).  IN FILE-TRANSFER

MODE YOU MAY SEND TO AND RECEIVE FILES FROM THE COMPUTER TO

WHICH YOU ARE CONNECTED.  LET'S BEGIN BY LEARNING ABOUT COM-

MAND MODE AND THE KERMIT PARSER.

COMMAND MODE

WHEN KERMIT FIRST BOOTS, YOU ARE PLACED AUTOMATICALLY IN

COMMAND MODE.  FROM HERE YOU CAN COMMUNICATE WITH THE PRO-

GRAM BY TYPING COMMANDS AT THE "KERMIT-65>" PROMPT.  COMMAND

MODE IS REALLY THE PROGRAM'S COMMAND PARSER, WHICH IS THAT

PART OF ANY COMPUTER PROGRAM WHICH TAKES YOUR COMMAND INPUT

AND TRIES TO INTERPRET IT AND THEN EXECUTE THE APPROPRIATE

ACTION.

KERMIT'S COMMAND PARSER IS VERY EASY TO USE.  LET'S BEGIN

WITH A LOOK AT THE MAIN LEVEL COMMANDS.  AT THE TOP OF THE

SCREEN YOU SHOULD SEE "TYPE ? FOR HELP".  SO LET'S DO JUST

THAT: TYPE A "?".  A LIST OF KERMIT'S MAIN COMMANDS (THERE

ARE 16 OF THEM) SHOULD APPEAR ON THE SCREEN IN TWO COLUMNS

LIKE THIS:

KERMIT-65>? KEYWORD, ONE OF THE FOLLOWING:

BYE         CONNECT

DIRECTORY   DISK

EXIT        FINISH

GET         HELP

QUIT        RECEIVE

RESTORE     SAVE

SEND        SET

SHOW        STATUS

THE "?" IS A VERY HELPFUL FEATURE OF THE COMMAND PARSER.

YOU CAN TYPE A "?" AT ANY POINT IN THE MIDDLE OF A COMMAND

TO RECEIVE A LIST OF THE VALID WAYS TO COMPLETE THE COMMAND.

A COMPLETE LIST OF COMMANDS MAY BE FOUND IN THE COMMAND

REFERENCE SECTION.

C-64/128 KERMIT USER'S GUIDE                          PAGE 5

LET'S TRY A SLIGHTLY MORE DIFFICULT COMMAND TO GIVE YOU

PRACTICE WITH THE PARSER.  FOR EXAMPLE, LET'S SAY WE WANT TO

CHANGE WHICH SCREEN DISPLAY WE ARE USING.  AFTER LOOKING AT

THE VARIOUS MAIN LEVEL COMMANDS, WE DEDUCE THAT THE SET COM-

MAND MIGHT BE THE WAY TO GET THE JOB DONE.  TYPE: SET FOL-

LOWED BY A SPACE AND THEN A "?".  KERMIT SHOULD RESPOND WITH

A LIST THAT LOOKS LIKE THIS:

KERMIT-65>SET ? KEYWORD, ONE OF THE FOLLOWING:

BAUD                 BOLD

BORDER               CHARACTER

DARK-BACKGROUND      DEBUGGING

EIGHT-BIT-QUOTING    ESCAPE

FILE-BYTE-SIZE       FILE-TYPE

FILE-WARNING         FLOW-CONTROL

IBM                  LIGHT-BACKGROUND

LOCAL-ECHO           PARITY

RECEIVE              RS232-REGISTERS

SEND                 SCREEN-DRIVER

TERMINAL-EMULATION   WORD-SIZE

AFTER LOOKING THROUGH THIS LIST, WE CAN DETERMINE WE NEED

THE SCREEN-DRIVER SUB-OPTION.  NOTICE THAT THE PARSER CON-

VENIENTLY REPRINTED THE SET COMMAND SO THAT WE CAN CONTINUE

WHERE WE LEFT OFF.  SO, TYPE: SCREEN-DRIVER FOLLOWED BY A

SPACE AND A "?".  NOW WE HAVE ANOTHER LIST, LOOKING LIKE

THIS:

KERMIT-65> SET SCREEN-DRIVER ? KEYWORD, ONE OF THE FOLLOWING:

40-COLUMNS     80-COLUMNS

BI-80          COMMODORE-128

CUSTOM-BI-80

IT SHOULD NOW BE OBVIOUS WHAT TO DO TO CHANGE TO A DIFFERENT

SCREEN DRIVER.  TRY IT OUT.  NOW, LET'S GO BACK TO 80

COLUMNS.

ONE THING YOU ARE PROBABLY WONDERING IS, "DO I HAVE TO TYPE

ALL OF THAT OUT?"  LUCKILY, NO, YOU DON'T.  THE LEFT ARROW

KEY (UPPER LEFT CORNER OF THE KEYBOARD) WILL SPELL OUT A

COMMAND ONE LEVEL AT A TIME FOR YOU IF YOU PROVIDE IT WITH

THE FIRST FEW CHARACTERS OF THE COMMAND.  TO TRY THIS OUT,

TYPE: SET SC AND HIT THE LEFT ARROW.  KERMIT SHOULD SPELL

OUT SCREEN-DRIVER AND PUT A SPACE AT THE END.  NOTICE THAT

WE HAD TO USE "SC" AND NOT JUST "S".  CONSULTING THE LIST

ABOVE, WE CAN SEE WHY.

BOTH SEND AND SCREEN-DRIVER BEGIN WITH AN "S".  IF WE HAD

JUST TYPED "S", KERMIT WOULD HAVE CHOSEN SEND BECAUSE IT IS

THE FIRST COMMAND IN THE LIST THAT MEETS THE SPECIFIED

REQUIREMENTS.  THE SAME APPLIES FOR WHY WE SPELLED OUT SET.

SEND IS BEFORE SET IN THE MAIN COMMAND LIST, MAKING THE LEFT

PAGE 6                          C-64/128 KERMIT USER'S GUIDE

ARROW USELESS.  SO KEEP IN MIND THAT YOU MUST ALWAYS USE

ENOUGH CHARACTERS TO UNIQUELY IDENTIFY THE COMMAND BEFORE

HITTING THE LEFT ARROW.  OK, LET'S FINISH OUR COMMAND.  TYPE

"8" AND HIT THE LEFT ARROW.  KERMIT SHOULD FINISH THE COM-

MAND.  NOW PRESS <RETURN>.

PRESTO!  WE'RE BACK TO 80-COLUMN MODE, SAVING 19 CHARACTER'S

WORTH OF TYPING.  NOT ALL COMMANDS ARE NEARLY THIS LONG, BUT

THE LEFT ARROW (ALONG WITH THE "?") CAN BE VERY HELPFUL.

THERE ARE A FEW OTHER USEFUL TRICKS TO KNOW ABOUT THE

PARSER.  THEY ARE:

1) PRESSING <CLR> OR <CTRL> - <L> INSTEAD OF <RETURN> TO

EXECUTE A COMMAND WILL CLEAR THE SCREEN FIRST AND THEN

EXECUTE THE COMMAND.

2) PRESSING <CTRL> - <W> WHILE TYPING A COMMAND WILL BACK

THE CURSOR UP TO THE PREVIOUS WORD, ERASING ANY ASSOCI-

ATED TEXT.

3) PRESSING <CTRL> - <U> WHILE TYPING A COMMAND WILL CAN-

CEL THE COMMAND BY BACKING THE CURSOR UP TO THE

"KERMIT-65>" PROMPT.

CONNECT MODE

CONNECT MODE IS THE MODE IN WHICH YOU WILL SPEND MOST OF

YOUR TIME WHILE USING KERMIT BECAUSE IT IS IN THIS MODE THAT

YOU CAN COMMUNICATE WITH ANOTHER COMPUTER (OFTEN KNOWN AS

THE REMOTE HOST).  NOW LET'S PREPARE TO CONNECT TO ANOTHER

COMPUTER.  TYPE: CONNECT.  THIS COMMAND WILL TAKE US OUT OF

COMMAND MODE AND INTO CONNECT MODE.  IF YOU TYPE CONNECT

THEN HIT <CLR>, THE SCREEN WILL BE CLEARED BEFORE ENTERING

CONNECT MODE.  EVERY TIME YOU ENTER CONNECT MODE, YOU WILL

SEE THE MESSAGE:

[CONNECTING TO HOST: TYPE ^Y C TO RETURN]

THIS TELLS US THAT WE ARE IN CONNECT MODE AND THAT TO RETURN

TO COMMAND MODE WE SHOULD TYPE <CTRL> - <Y> THEN TYPE "C".

TRY IT OUT.  THEN RETURN TO CONNECT MODE.

NOW WE WILL CALL THE OTHER COMPUTER.  A COMMODORE 1670 OR

OTHER HAYES COMPATIBLE MODEM CAN DIAL DIRECTLY FROM CONNECT

MODE VIA ITS BUILT-IN DIALING COMMANDS, BUT WITH MOST OTHER

MODEMS YOU WILL HAVE TO DIAL MANUALLY.  IN OTHER WORDS, KER-

MIT DOES NOT HAVE AN AUTO-DIAL COMMAND.  ALSO PLEASE NOTE

THAT KERMIT CURRENTLY DOES NOT WORK WITH THE COMMODORE 1660

MODEM.  IT WILL, HOWEVER, BE SUPPORTED IN A FUTURE VERSION.

PLEASE SEE APPENDIX A: IMPLEMENTATION NOTES FOR FURTHER

INFORMATION.

C-64/128 KERMIT USER'S GUIDE                          PAGE 7

WE NEED TO CHECK A FEW THINGS BEFORE DIALING.  EACH COMPUTER

YOU CONNECT TO MAY HAVE DIFFERENT BAUD, PARITY, AND WORD-

SIZE REQUIREMENTS.  KERMIT DEFAULTS TO 1200 BPS TRANSMIS-

SION, EIGHT BIT WORD LENGTH, NO PARITY, AND VT-100 TERMINAL

EMULATION.  TO ADJUST ANY OF THESE PARAMETERS, ESCAPE BACK

TO COMMAND MODE AND USE THE SET COMMAND AS WE DID ABOVE TO

CHANGE THE SCREEN-DRIVER. FOR EXAMPLE, TO CHANGE TO 300 BPS

TRANSMISSION SPEED, TYPE: SET BAUD 300.

IF YOU WISH TO SAVE ANY CHANGES YOU MAKE AS NEW DEFAULTS,

PLEASE SEE THE SAVE COMMAND EXPLANATION IN THE COMMAND

REFERENCE SECTION.

ONCE YOU HAVE SET UP YOUR COMMUNICATIONS PARAMETERS

CORRECTLY, ALL THAT'S LEFT TO DO IS DIAL THE PHONE AND CON-

NECT TO THE REMOTE HOST.

FILE-TRANSFER MODE

ONCE YOU HAVE CONNECTED TO A REMOTE HOST YOU MAY WISH TO

SEND AND/OR RECEIVE FILES.  WITH KERMIT YOU CAN SEND AND

RECEIVE MOST ANY KIND OF FILE, AS KERMIT HAS AUTOMATIC

TRANSLATION FEATURES FOR MANY TYPES OF FILES.  FOR EXAMPLE,

YOU CAN AUTOMATICALLY CONVERT A PETSCII (COMMODORE'S SPECIAL

ASCII) TEXT FILE TO ASCII WHEN SENDING IT TO ANOTHER

MACHINE, AND VICE VERSA.  FOR A LIST OF THE VARIOUS FILE-

TYPES SUPPORTED, SEE THE SET FILE-TYPE COMMAND EXPLANATION

IN THE COMMAND REFERENCE SECTION.

THE PROCEDURES FOR TRANSFERRING FILES WITH KERMIT CAN DIFFER

WIDELY DEPENDING ON THE SYSTEM TO WHICH YOU ARE CONNECTED.

THIS TUTORIAL CAN'T POSSIBLY COVER EVERY POSSIBLE SITUATION.

INSTEAD, WE WILL WALK THROUGH A SIMPLE TRANSFER AND POINT

OUT SOME THINGS OF WHICH YOU SHOULD BE AWARE.  THE MOST

IMPORTANT THING WHEN ATTEMPTING TO TRANSFER FILES WITH KER-

MIT IS TO HAVE YOUR FILE-TRANSFER PARAMETERS SET CORRECTLY!

YOU MAY WANT TO SKIP AHEAD TO THE TRANSFERRING FILES SECTION

FOR MORE INFORMATION.

NOW, WE WILL SHOW TWO EXAMPLES: 1) UPLOADING A PETSCII

SEQUENTIAL TEXT FILE, CONVERTING IT TO STANDARD ASCII, AND

2) DOWNLOADING A C LANGUAGE PROGRAM SOURCE CODE FILE FOR USE

WITH THE C POWER (NOW SOLD AS POWER C) C COMPILER FOR THE

64/128.

TEXT FILE:

WE ARE SENDING AND THE OTHER END IS RECEIVING.  IN NOTE

FORM, HERE IS WHAT WE NEED TO DO:

1) CHECK FILE-TRANSFER PARAMETERS!!  MAKE SURE THE FILE-

TYPE IS SET TO PETSCII.

PAGE 8                          C-64/128 KERMIT USER'S GUIDE

2) START KERMIT ON THE HOST MACHINE USING THE APPROPRIATE

COMMAND.

3) TELL THE HOST KERMIT TO RECEIVE THE FILE.

4) ESCAPE TO COMMAND MODE IN OUR KERMIT (<CTRL> - <Y> THEN

<"C">).  TYPE SEND FOLLOWED BY THE FILENAME OF THE COM-

MODORE TEXT FILE YOU WISH TO SEND.

WHEN THE TWO KERMITS HAVE FINISHED TRANSFERRING THE FILE,

YOU WILL BE RETURNED TO COMMAND MODE.  TYPE: CONNECT AND

CONTINUE WITH YOUR LOGIN SESSION.

NOTE: THE RECOMMENDED PARAMETERS (WORK MOST OF THE TIME)

ARE:

FILE TYPE IS PETSCII

FILE BYTE SIZE IS EIGHT-BIT (IF POSSIBLE WITH THE HOST

SYSTEM)

EVERYTHING ELSE SHOULD RETAIN ITS DEFAULT VALUE.

C POWER FILE:

WE ARE RECEIVING AND THE HOST IS SENDING.  IN NOTE FORM,

HERE IS WHAT WE NEED TO DO:

1) CHECK PARAMETERS!!  MAKE SURE THE FILE-TYPE IS SET TO

C-POWER.

2) START KERMIT ON THE HOST MACHINE USING THE APPROPRIATE

COMMAND.

3) TELL THE HOST KERMIT TO SEND THE FILE.

4) ESCAPE TO COMMAND MODE IN OUR KERMIT (<CTRL> - <Y> THEN

<"C">).  TYPE RECEIVE FOLLOWED BY THE FILENAME OF THE C

SOURCE CODE FILE YOU WISH TO RECEIVE IN C POWER FORMAT.

5) WHEN THE TWO KERMITS HAVE FINISHED TRANSFERRING THE

FILE, YOU WILL BE RETURNED TO COMMAND MODE.  TYPE: CON-

NECT AND CONTINUE WITH YOUR LOGIN SESSION.

NOTE: THE RECOMMENDED PARAMETERS (WORK MOST OF THE TIME)

ARE:

FILE TYPE IS C-POWER

FILE BYTE SIZE IS EIGHT-BIT

EIGHT BIT QUOTING IS ON

PARITY IS NONE (IF POSSIBLE WITH THE HOST SYSTEM)

WORD SIZE IS EIGHT-BIT (IF POSSIBLE WITH THE HOST SYSTEM)

C-64/128 KERMIT USER'S GUIDE                          PAGE 9

EVERYTHING ELSE SHOULD RETAIN ITS DEFAULT VALUE.

IF YOU HAVE PERSISTENT PROBLEMS WITH TRANSFERRING FILES,

DON'T HESITATE TO ASK SOMEONE FOR HELP.  ALSO TRY VERBOSE

DEBUGGING TO SEE MORE OF WHAT IS HAPPENING DURING TRANSFERS.

KERMIT IS VERY FLEXIBLE, BUT FLEXIBLE CAN MEAN COMPLICATED!

WE NOW HAVE COMPLETED THE INTRODUCTORY TUTORIAL. THE NEXT

SECTION OF THIS MANUAL, TROUBLESHOOTING, MAY HELP IF YOU

HAVE SOME PROBLEMS USING KERMIT.  THE COMMAND REFERENCE SEC-

TION DESCRIBES IN DETAIL EACH OF KERMIT'S COMMANDS AND WILL

PROVE TO BE INVALUABLE TO SUCCESSFULLY USING THIS PROGRAM.

GOOD LUCK WITH KERMIT!