HCD65XX is a powerful macro assembler syntactically identical

to the assembler used to orginally develop the C128 source code running

on a VAX-8600 under VMS. This tool is capable of assembling the same

source files on a C128.

HCD65XX consists of a small basic shell, and a large machine

code program. The basic program is used as the interface to the user

allowing him to easily configure the assemebler for various tasks.

The machine language portion of HCD65XX is set up by the basic

program, performs the actual assembly, and returns control to BASIC

when the assembly is completed. Because the user interface is

in BASIC, it may be easily customized.

The user has control over the following parameters:

list file channel

list channel width

object file channel

error channel

source file name

minimum source file unit

maximum source file unit

cross reference file unit

date shown on listing

The list file channel is the logical file number the assembler

will use to write the list file too. The basic program first

opens up a channel, and then informs the assembler of which channel

to use. In this way the listing output may be directed to the

printer, the screen, rs232, or to disk. The assembler does not

support cassette operations. If the user assigns this channel a

value of zero, then the list file is inhibited.

The list channel width is used to control the maximum line length

output by the assembler. This is so that both 80 coloumn and

132 coloumn printers can both get the most out of the system.

The object file channel operates much in the same way as the

list file channel. The basic program opens a file or device,

and the program sends it there. Here again the object file

may be inhibited.

The error channel is special. Errors always show up in the

listing file. However it is possible to have those errors

also broadcast out via the error channel. This you can

direct the list file to disk and the errors to the screen.

UNIT ALLOCATION:

note:	secondary addresses on disk drives are assuemed to

be the same value as the unit number. this prevents

the internal operation from comflicting with

disk channels established by the users basic program.

0,1	reserved

1,2,3,4,5,6,7	usable for users direction of list,obj,error channels

8,9,10,11,12,13,14,15	reserved for interal disk operations.

8-11	nested includes