C-64/128 KERMIT USER'S GUIDE                         PAGE 17

TRANSFERRING FILES

TRANSFERRING FILES WITH KERMIT ISN'T REALLY ANY MORE DIFFI-

CULT THAN WITH MOST OTHER COMMODORE TELECOMMUNICATIONS

PACKAGES/PROTOCOLS.  IN FACT, KERMIT IS MUCH MORE FLEXIBLE--

BUT THAT'S WHERE CONFUSION BEGINS.  THERE ARE THREE MAJOR

PARAMETERS TO BE CONCERNED WITH WHEN TRANSFERRING FILES.

THEY ARE: 1) FILE-TYPE, 2) FILE-BYTE-SIZE, AND 3) EIGHT-

BIT-QUOTING.  LET'S DISCUSS EACH OF THEM IN TURN, BEGINNING

WITH FILE-TYPE.

CHOOSING THE CORRECT FILE-TYPE

KERMIT CURRENTLY SUPPORTS FIVE DIFFERENT FILE TYPES. THEY

ARE:

A) ASCII

B) BINARY

C) C-POWER

D) PETSCII

E) SCRIPT

EACH OF THESE FILE TYPES HAS A DISTINCT USE.  WHEN TO USE

ONE FILE-TYPE OVER ANOTHER CAN BE CONFUSING INITIALLY.  SO,

WE'LL DISCUSS EACH OF THEM SEPARATELY, STARTING WITH ASCII.

ASCII FILE-TYPE

THIS FILE-TYPE IS USED A LOT LESS OFTEN THAN YOU MIGHT THINK

AT FIRST.  BECAUSE COMMODORE COMPUTERS (EXCEPT FOR THE AMI-

GAS AND PCS) USE THEIR OWN, MODIFIED CHARACTER SET ARRANGE-

MENT (PETSCII), MOST TEXT FILES YOU TRANSFER TO AND FROM

YOUR COMMODORE WILL BE PETSCII FILES, NOT STANDARD ASCII.

YOU WILL USE THIS FILE-TYPE ONLY WHEN YOU A) WISH TO DOWN-

LOAD A STANDARD ASCII FILE AND NOT CONVERT IT TO A FORM

USABLE ON YOUR COMMODORE (PETSCII), OR B) YOU HAVE, FOR SOME

REASON, A STANDARD ASCII FILE THAT YOU WISH TO UPLOAD TO

ANOTHER COMPUTER.

IF YOU HAPPEN TO CHOOSE THIS FILE-TYPE WHEN YOU REALLY

NEEDED PETSCII, YOU WLL BE ABLE TO TELL IMMEDIATELY, AS THE

CASE OF ALL THE TEXT WILL BE REVERSED.

BINARY FILE-TYPE

THE BINARY FILE-TYPE IS NOT USED VERY OFTEN.  THIS FILE-TYPE

IS USED TO TRANSFER COMMODORE PROGRAMS THAT ARE EXECUTABLE

(I.E. A BASIC PROGRAM OR A MACHINE LANGUAGE GAME, OR EVEN

KERMIT).  IN OTHER, MORE TECHNICAL, TERMS, THIS FILE-TYPE

ALLOWS YOU TO TRANSFER FILES THAT HAVE FULL EIGHT-BIT DATA

WORDS.  TEXT FILES, BY DEFINITION, CONTAIN ONLY SEVEN-BIT

PAGE 18                         C-64/128 KERMIT USER'S GUIDE

DATA WORDS.  NOTE THAT IT'S GENERALLY A GOOD IDEA TO HAVE

EIGHT-BIT-QUOTING SET ON WHEN TRANSFERRING BINARY FILES, BUT

MORE ABOUT THAT BELOW.

C-POWER FILE-TYPE

THIS FILE-TYPE IS DESIGNED SPECIFICALLY TO TRANSFER SOURCE

CODE FILES FOR USE WITH THE C POWER (NOW POWER C) C COMPILER

FOR THE C-64 AND C-128.  C POWER FILES HAVE, IN ADDITION TO

THE STANDARD PETSCII FILE-TYPE BASE, A FEW RE-DEFINED CHAR-

ACTERS THAT ARE NEEDED FOR C PROGRAMMING, SUCH AS BRACES,

THE UNDERSCORE, THE PIPE, ETC.

WHEN YOU USE THIS FILE-TYPE, ANY STANDARD C SOURCE CODE

FILES YOU DOWNLOAD TO YOUR COMMODORE WILL BE CONVERTED TO C

POWER FORMAT.  ON THE OTHER SIDE OF THE COIN, ANY C POWER

FILES YOU UPLOAD TO ANOTHER COMPUTER WILL BE CONVERTED TO

STANDARD ASCII SOURCE CODE FILES.

PETSCII FILE-TYPE

THIS FILE-TYPE IS THE MOST COMMON FILE-TYPE, AND SO IT IS

THE DEFAULT KERMIT FILE-TYPE.  WHEN YOU USE THIS FILE-TYPE,

ANY COMMODORE TEXT FILE YOU UPLOAD TO ANOTHER COMPUTER WILL

BE CONVERTED TO STANDARD ASCII TEXT.  GOING THE OTHER DIREC-

TION, WHEN YOU DOWNLOAD A STANDARD ASCII TEXT FILE TO YOUR

COMMODORE, IT WILL BE TRANSLATED TO PETSCII.

IF YOU CHOOSE THIS FILE TYPE WHEN YOU REALLY MEANT ASCII,

THE CASE OF ALL THE LETTERS IN THE TEXT WILL BE REVERSED.

SCRIPT FILE-TYPE

THIS IS A SPECIAL FILE-TYPE FOR USE WHEN TRANSFERRING FILES

FOR THE COMMODORE SPEEDSCRIPT WORD PROCESSOR (COMPUTE! PUB-

LICATIONS).  SPEEDSCRIPT FILES HAVE A SPECIAL FILE FORMAT.

IF YOU WANT TO DOWNLOAD A STANDARD TEXT FILE FROM ANOTHER

COMPUTER FOR SPEEDSCRIPT, YOU MUST USE THIS FORMAT.  THE

SAME APPLIES FOR SENDING A SPEEDSCRIPT FILE TO ANOTHER COM-

PUTER THAT USES STANDARD ASCII TEXT FILES.

SETTING THE FILE-BYTE-SIZE

THIS PARAMETER IS OFTEN CONFUSED BY KERMIT USERS BECAUSE IT

SOUNDS A LOT LIKE THE WORD-SIZE PARAMETER.  THESE TWO PARAM-

ETERS ARE ACTUALLY QUITE DIFFERENT.  WHEREAS WORD-SIZE

DESCRIBES THE DATA PATH, FILE-BYTE-SIZE DESCRIBES THE SIZE

OF THE DATA WORDS WITHIN FILES BEING TRANSFERRED.

THIS PARAMETER CAN ALWAYS BE LEFT SET TO EIGHT-BIT.  TECHNI-

CALLY, THOUGH, IT IS REALLY ONLY NECESSARY TO BE SET TO

C-64/128 KERMIT USER'S GUIDE                         PAGE 19

EIGHT-BIT WHEN YOU ARE TRANSFERRING BINARY OR PETSCII FILES.

USING EIGHT-BIT-QUOTING

THIS FEATURE ALLOWS YOU TO TRANSFER BINARY OR PETSCII FILES

TO AND FROM COMPUTERS THAT WOULD NORMALLY BE UNABLE TO HAN-

DLE THEM.  EIGHT-BIT-QUOTING ALLOWS YOU SEND FILES WITH

EIGHT-BIT DATA WORDS OVER A SEVEN-BIT DATA PATH.  THIS

PARAMETER CAN ALWAYS BE LEFT SET TO ON.

FOR INSTANCE, I OFTEN COMMUNICATE WITH A MACHINE WHERE MY

COMMUNICATIONS PARAMETERS HAVE TO BE WORD-SIZE OF SEVEN-BIT

AND EVEN PARITY.  IN THIS KIND OF ENVIRONMENT I WOULD NOR-

MALLY BE UNABLE TO SEND A PROGRAM FILE TO A FRIEND ON THAT

MACHINE, AS PROGRAM FILES ARE BINARY (EIGHT-BITS).  BUT IF I

MAKE SURE EIGHT-BIT-QUOTING IS ON, EVERYTHING WILL BE FINE.

TECHNICALLY, EIGHT-BIT-QUOTING PREFIXES EACH BYTE OF DATA

THAT IS OVER SEVEN BITS LONG (GREATER THAN ASCII 127 DECIMAL

IN VALUE) SO THAT IT CAN BE TRANSFERRED ALONG THE SEVEN-BIT

DATA PATH.

SPECIAL FILE-TRANSFER MODE FEATURES

THERE ARE THREE HELPFUL FEATURES AVAILABLE WHEN TRANSFERRING

FILES.  THESE FEATURES ARE STANDARD IN MOST VERSIONS OF KER-

MIT.  THEY ARE:

KEY SEQUENCE   ACTION

<RETURN>       FORCE REMOTE KERMIT TO RE-SEND CURRENT PACKET

<CTRL> - <X>   CANCEL TRANSMISSION OF CURRENT FILE

<CTRL> - <Y>   CANCEL TRANSMISSION OF ENTIRE BATCH OF FILES

ALL OF THESE COMMANDS ARE AVAILABLE WHEN RECEIVING A FILE OR

FILES.  BECAUSE C-64/128 KERMIT CAN'T SEND MORE THAN ONE

FILE PER SEND COMMAND (A FEATURE CALLED "WILDCARD SEND"),

THE <CTRL> - <Y> FEATURE IS NOT AVAILABLE WHEN SENDING A

FILE.

PAGE 20                         C-64/128 KERMIT USER'S GUIDE

COMMAND REFERENCE

COMMAND LIST

KERMIT V2.1 HAS 16 MAJOR COMMANDS, MOST OF WHICH HAVE SUB-

OPTIONS.  BELOW IS A LIST OF THE MAJOR COMMANDS.

BYE    CONNECT   DIRECTORY   DISK      EXIT      FINISH

GET    HELP      QUIT        RECEIVE   RESTORE   SAVE

SEND   SET       SHOW        STATUS

COMMAND SUMMARY

EACH OF THE MAJOR COMMANDS WILL NOW BE DESCRIBED IN ALPHA-

BETICAL ORDER.  FOLLOWING THE DESCRIPTION IS ONE OR MORE

EXAMPLES USING THE COMMAND PREVIOUSLY DESCRIBED.  PLEASE

NOTE THAT ALL COMMANDS MAY BE ENTERED IN EITHER LOWER OR

UPPER CASE.  CASE DISTINCTION IS NOT MADE BY THE KERMIT

PARSER.

BYE

THIS COMMAND WILL CAUSE C-64/128 KERMIT TO TELL A SERVER

KERMIT AT THE OTHER END OF THE CONNECTION TO EXIT FROM KER-

MIT AND, IF APPLICABLE, TERMINATE ITS JOB (OR PROCESS,

ETC.).  BYE HAS NO ARGUMENTS, WITH THE FORMAT:

KERMIT-65>BYE

CONNECT

THE CONNECT COMMAND PUTS KERMIT INTO CONNECT MODE (SEE THE

INTRODUCTORY TUTORIAL SECTION FOR MORE INFORMATION ON CON-

NECT MODE).  KERMIT CAN THEN BEGIN COMMUNICATION WITH

ANOTHER COMPUTER AS A TERMINAL (AS DEFINED BY THE SET

TERMINAL-EMULATION COMMAND.  CONNECT HAS NO ARGUMENTS, WITH

THE FORMAT:

KERMIT-65>CONNECT

TO RETURN TO COMMAND MODE (SEE THE INTRODUCTORY TUTORIAL

SECTION FOR MORE INFORMATION ON COMMAND MODE), TYPE THE

ESCAPE CHARACTER FOLLOWED BY A <"C">.  THE DEFAULT ESCAPE

CHARACTER IS <CTRL> - <Y>.  THE ESCAPE CHARACTER MAY BE FOL-

LOWED BY CHARACTERS OTHER THAN <"C">.  THE OTHER VALID

OPTIONS AND WHAT THEY DO CAN BE VIEWED BY SUBSTITUTING A

<"?"> FOR THE <"C">, GIVING THE DISPLAY:

C-64/128 KERMIT USER'S GUIDE                         PAGE 21

ONE OF THE FOLLOWING:

?             - THIS HELP MESSAGE.

B             - SEND A BREAK SIGNAL.

C             - CLOSE THE CONNECTION.

S             - STATUS OF CONNECTION.

0             - SEND A NULL.

ESCAPE-CHAR   - TRANSMIT THE ESCAPE CHARACTER.

DIRECTORY

THE DIRECTORY COMMAND WILL RETURN A DISK DIRECTORY OF THE

SPECIFIED FILES ON DEVICE 8 (THE NORMAL DEVICE NUMBER FOR

THE FIRST SERIAL COMMODORE DISK DRIVE).  DIRECTORY WITH NO

ARGUMENTS RETURNS A COMPLETE DISK DIRECTORY.  THE DIRECTORY

MAY BE PAUSED BY PRESSING <SPACE> AND RESUMED BY PRESSING

ANY KEY, AND ABORTED BY PRESSING <STOP>.  THE FORMAT IS:

KERMIT-65>DIRECTORY [FILENAME]

WHERE [FILENAME] IS A VALID CBM DOS FILENAME.  SOME EXAM-

PLES:

KERMIT-65>DIRECTORY        (DIRECTORY OF ALL FILES

ON DEVICE 8)

KERMIT-65>DIRECTORY KER*   (DIRECTORY OF FILES BE-

GINNING WITH "KER")

KERMIT-65>DIRECTORY K?W    (DIRECTORY OF FILES 3

CHARACTERS LONG, BEGIN-

NING WITH "K" AND ENDING

WITH "W")

DISK

THE DISK COMMAND WILL ALLOW YOU TO SEND DOS COMMANDS

DIRECTLY TO THE DISK DRIVE CONTROLLER.  THIS COMMAND IS USE-

FUL FOR INITIALIZING THE DRIVE, FORMATTING A DISK, AND

DELETING UNWANTED FILES.  THIS COMMAND SUPPORTS ALL THE

STANDARD COMMODORE DOS COMMANDS, EXCEPT THAT NO SPACES IN

FILENAMES ARE ALLOWED.  DISK WITH NO ARGUMENTS WILL RETURN

THE DRIVE ERROR IN STANDARD DOS FORMAT IF THE DRIVE'S RED

LIGHT IS BLINKING.  THE FORMAT IS:

KERMIT-65>DISK [STRING]

WHERE [STRING] IS A VALID CBM DOS COMMAND.  SOME EXAMPLES:

PAGE 22                         C-64/128 KERMIT USER'S GUIDE

KERMIT-65>DISK S0:K*         (DELETE ALL FILES BEGIN-

NING WITH "K" ON DEVICE

8)

KERMIT-65>DISK I0:           (INITIALIZE DEVICE 8)

KERMIT-65>DISK N0:FILES,01   (FORMAT DISK IN DEVICE 8,

GIVING IT NAME "FILES"

AND ID "01"-- NOTE THE

LACK OF SPACES IN THE

DISK NAME CHOSEN)

EXIT

THE EXIT COMMAND WILL CAUSE C-64/128 KERMIT TO STOP EXECU-

TION AND RETURN CONTROL OF THE MACHINE TO BASIC.  THIS COM-

MAND IS THE SAME AS THE QUIT COMMAND.  KERMIT MAY BE RES-

TARTED IMMEDIATELY WITH THE COMMAND RUN.  IF YOU TYPE NEW,

KERMIT CAN STILL BE RESTARTED MOST OF THE TIME WITH THE COM-

MAND SYS 2064.  EXIT HAS NO ARGUMENTS.  THE FORMAT OF THIS

COMMAND IS:

KERMIT-65>EXIT

FINISH

THIS COMMAND WILL CAUSE KERMIT TO TELL A SERVER KERMIT AT

THE OTHER END OF THE CONNECTION TO EXIT.  WHEN C-64/128 KER-

MIT RECEIVES THE ACKNOWLEDGEMENT THAT THIS IS BEING DONE,

YOU WILL BE PROMPTED FOR ANOTHER COMMAND.  FINISH HAS NO

ARGUMENTS.  THE FORMAT OF THIS COMMAND IS:

KERMIT-65>FINISH

GET

THIS COMMAND WILL CAUSE C-64/128 KERMIT TO REQUEST THAT THE

OTHER KERMIT (WHICH MUST BE RUNNING IN SERVER MODE) TO

TRANSMIT THE SPECIFIED FILE TO THE C-64/128 KERMIT.  THE

FORMAT OF THIS COMMAND IS:

KERMIT-65>GET [FILE-SPECIFICATION]

WHERE [FILE-SPECIFICATION] IS A VALID FILE SPECIFICATION FOR

THE SYSTEM ON WHICH THE OTHER KERMIT IS RUNNING.

QUIT

THIS COMMAND WILL CAUSE C-64/128 KERMIT TO RETURN TO THE

BASIC COMMAND LEVEL.  QUIT IS IDENTICAL TO THE EXIT COMMAND

(SEE EXIT ABOVE FOR MORE INFORMATION).  QUIT HAS NO ARGU-

MENTS, WITH THE FORMAT: