C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 1

This document details the many user-callable routines

available in the  C128  BASIC  7.0 math package.  The

C128  represents  the  first Commodore 65xx system to

fully  vectorize and  describe the calling parameters

necessary  to  fully and safely utilize the available

math subroutines.  The  table  below  summarizes  the

documented routines and  provides the location of the

jump vector in the BASIC 7.0 ROMs.

FRED BOWEN		APRIL 22, 1986

Format Conversions

AF00	AYINT		;convert f.p. to integer

AF03	GIVAYF		;convert integer to f.p.

AF06	FOUT		;convert f.p. to ascii string

AF09	VAL_1		;convert ascii string to f.p.

AF0C	GETADR		;convert f.p. to an address

AF0F	FLOATC		;convert address to f.p.

Math Functions

AF12	FSUB		;MEM - FACC

AF15	FSUBT		;ARG - FACC

AF18	FADD		;MEM + FACC

AF1B	FADDT		;ARG + FACC

AF1E	FMULT		;MEM * FACC

AF21	FMULTT		;ARG * FACC

AF24	FDIV		;MEM / FACC

AF27	FDIVT		;ARG / FACC

AF2A	LOG		;natural logarithm of FACC

AF2D	INT		;greatest integer of FACC

AF30	SQR		;square root of FACC

AF33	NEGOP		;negate (invert sign) of FACC

AF36	FPWR		;ARG to the MEM power

AF39	FPWRT		;ARG to the FACC power

AF3C	EXP		;EXP of FACC

AF3F	COS		;COS of FACC

AF42	SIN		;SIN of FACC

AF45	TAN		;TAN of FACC

AF48	ATN		;ATN of FACC

AF4B	ROUND		;round FACC

AF4E	ABS		;absolute value of FACC

AF51	SIGN		;sign of FACC

AF54	FCOMP		;compare FACC with MEM

AF57	RND_0		;generate random number in FACC

Movement

AF5A	CONUPK		;move RAM MEM to ARG

AF5D	ROMUPK		;move ROM MEM to ARG

AF60	MOVFRM		;move RAM MEM to FACC

AF63	MOVFM		;move ROM MEM to FACC

AF66	MOVMF		;move FACC to MEM

AF69	MOVFA		;move ARG to FACC

AF6C	MOVAF		;move FACC to ARG

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 2

Floating Point Math Package Conventions

In BASIC memory the number is PACKED and looks like this:

1       2       3       4       5

-----------------------------------------

|signed	|B7=SGN |	|	|	|

|  EXP	|-------|M A N T I S S A|  LSB	|

| +$80	|  MSB	|	|	|	|

-----------------------------------------

Because the mantissa  is normalized such that its msb is always

1, BASIC stores the SIGN of the mantissa here to save a byte of

storage. It must be normalized when put in the FACC (see CONUPK).

In the FACC the NORMALIZED number looks like this:

$63     $64     $65     $66     $67     $68

FACEXP  FACHO   FACMOH  FACMO   FACLO   FACSGN

-------------------------------------------------

|signed	|BIT 7=1|	|	|	| SIGN	|

|  EXP	|-------|M A N T I S S A|  LSB	|+ = $00|

| +$80	|  MSB	|	|	|	|- = $FF|

-------------------------------------------------

Negative exponents are not stored 2's complement.  The  maximum

exponent is 10^38 ($FF) and the minimum is 10^-39 ($01). A zero

value for the  exponent  means  the number is zero.  Since  the

exponent is a power of 2,  it can be described as the number of

left (EXP>$80)  or right  (EXP<=$80)  shifts to be performed on

the normalized  mantissa to create the binary representation of

the value.  There is a second  floating accumulator called  ARG

which has the same layout.  It  is  located at $6A through $6F.

Throughout the math package the floating point format is:

* the mantissa is 24 bits long.

* the binary point is to the left of the msb.

* the mantissa is always positive, and its msb is always 1.

* number = mantissa * 2^exponent, sign in FACSGN.

* the sign of the exponent is the msb of the exponent.

* the exponent is stored in excess $80 (i.e., it is a signed

8-bit number with $80 added to it.)

* an exponent of zero means the number is zero. (note that the

rest of the accumulator cannot be assumed to be zero.)

* to keep the same number in the accumulator while shifting:

right shifts --> increment exponent

left shifts  --> decrement exponent

Arithmetic routine calling conventions:

* For one argument functions:

the argument is in the FACC.

the result is left in the FACC.

* For two argument operations:

the first argument is in MEMORY (packed) or ARG (unpacked).

the second argument is in the FACC.

the result is left in the FACC.

* Always call ROM routines with the SYSTEM memory configuration

in context ($FF00=$00, ROMs, RAM-0, I/O) except as noted herein.

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 3

A note concerning precision.  Since  the  mantissa  is  always

normalized, the high order bit of the most significant byte is

always one.  This guarantees at least 40 bits (5 byte mantissa

times 8 bits each)  of  precision,  which  is  approximately 9

significant  digits  plus  a few bits for rounding.  In  fact,

there is a 'rounding' byte, FACOV ($71), which should, for the

greatest degree of precision,  be loaded whenever you load the

FACC.  The high order bit of FACOV is utilized in most  of the

math routines.  While some of the 'movement' routines  'round'

the loaded floating point number  (i.e., FACOV = $00),  others

(such as CONUPK) do not- assuming the value of  FACOV  is  the

useful result of an operation in progress. In 99% of the cases

you need not worry about it,  as its significance is virtually

nil. For the greatest degree of precision however, use it.

A few examples of normalized (FACC) floating point numbers:

VALUE		EXP	     M A N T I S S A	       SIGN

-------	       -----	--------------------------     ----

1E38	=	FF	96	76	99	53	00

4E10	=	A4	95	02	F9	00	00

2E10	=	A3	95	02	F9	00	00

1E10	=	A4	95	02	F9	00	00

10	=	84	A0	00	00	00	00

1	=	81	80	00	00	00	00

.5	=	80	80	00	00	00	00

.25	=	7F	80	00	00	00	00

.6	=	80	99	99	99	9A	00

1E-04	=	73	D1	B7	59	59	00

1E-37	=	06	88	1C	EA	15	00

1E-38	=	02	D9	C7	DC	EE	00

3E-39	=	01	82	AB	1E	2A	00

0	=	00	xx	xx	xx	xx	00

-1	=	81	80	00	00	00	FF

-5	=	83	A0	00	00	00	FF

Now for a simple example of deriving the actual binary from the FACC:

5	=	83	A0	00	00	00	00

|	  \

|	   \

($83-$80)	  ($A0)

|	    |

which means:   2^3   *  .10100000,  or shift mantissa LEFT 3,

which gives:	101.00000 (binary) or 5.0 (hex)

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 4

NAME:		AYINT	($AF00)

FUNCTION:	CONVERT FLOATING POINT TO INTEGER

PREPARATION:	FACC contains floating point number (-32768<=n<=32767)

RESULT:		FACMO ($66) contains signed integer (msb)

FACLO ($67) contains signed integer (lsb)

ERROR:		?ILLEGAL QUANTITY ERROR if FACC too big.

EXAMPLE:	JSR $AF00	;INT(FACC)

LDA $66		;MSB

LDY $67		;LSB

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		GIVAYF	($AF03)

FUNCTION:	CONVERT INTEGER TO FLOATING POINT

PREPARATION:	.A contains signed integer (msb)

.Y contains signed integer (lsb)

RESULT:		FACC contains floating point number

EXAMPLE:	LDA #>INTEGER

LDY #<INTEGER

JSR $AF03	;FLOAT (A,Y)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FOUT	($AF06)

FUNCTION:	CONVERT FLOATING POINT TO ASCII STRING

PREPARATION:	FACC contains floating point number

RESULT:		FBUFFR ($100) contains ASCII string (null terminated)

.A contains pointer to string (lsb)

.Y contains pointer to string (msb)

EXAMPLE:	JSR $AF06	;CONVERT FACC TO STRING AT $100

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 5

NAME:		VAL_1	($AF09)

FUNCTION:	CONVERT ASCII STRING TO FLOATING POINT

PREPARATION:	INDEX1 ($24,$25) contains pointer to string

.A contains length of string

SPECIAL NOTES:	String *must* be in RAM-1 or common RAM.  Any

invalid character terminates conversion when

encountered (i.e., acts like a terminator).

***********************************************

*  *MUST*  be called from ROM or common RAM-  *

*  this routine RTS's with RAM-1 in context!  *

***********************************************

RESULT:		FACC contains floating point number

EXAMPLE:	LDA #<POINTER

LDY #>POINTER

STA INDEX1	;SET POINTER TO STRING

STY INDEX1+1

LDA #LENGTH	;SET STRING LENGTH

JSR $AF09	;FACC = VAL(STRING)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		GETADR	($AF0C)

FUNCTION:	CONVERT FLOATING POINT TO ADDRESS

PREPARATION:	FACC contains floating point number (0<=n<=65535)

RESULT:		POKER ($16,$17) contains unsigned integer address

ERROR:		?ILLEGAL QUANTITY ERROR if FACC too big.

EXAMPLE:	JSR $AF0C	;ADR(FACC)

LDA $16		;LSB

LDY $17		;MSB

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FLOATC	($AF0F)

FUNCTION:	CONVERT ADDRESS TO FLOATING POINT

PREPARATION:	FACHO  ($64) contains address (msb)

FACMOH ($65) contains address (lsb)

.X   contains exponent ($90 always)

.C=1 if positive (always)

RESULT:		FACC contains floating point number

ERROR:		?OVERFLOW ERROR if FACC too big.

EXAMPLE:	LDA #<ADDRESS

LDY #>ADDRESS

STA FACMOH	;SET ADDRESS

STY FACHO

LDX #$90	;EXPONENT

SEC		;POSITIVE

JSR $AF0F	;FLOAT ADDRESS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 6

NAME:		FSUB	($AF12)

FUNCTION:	FACC = MEMORY - FACC

PREPARATION:	FACC contains floating point subtrahend

.A = pointer (lsb) to packed floating point minuend

.Y = pointer (msb) to packed floating point minuend

SPECIAL NOTES:	The minuend *MUST* be in RAM-1 or common RAM in packed

format. FSUB calls CONUPK to normalize it.

RESULT:		FACC contains floating point difference

ERROR:		?OVERFLOW ERROR if FACC too big.

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* MINUEND

JSR $AF12	;SUBTRACT MEMORY FROM FACC, DIFF IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FSUBT	($AF15)

FUNCTION:	FACC = ARG - FACC

PREPARATION:	FACC contains floating point subtrahend

ARG  contains floating point minuend

SPECIAL NOTES:	This routine is similar to FSUB. The only difference

is the call to CONUPK- FSUBT assumes you have already

loaded ARG with unpacked minuend.)

RESULT:		FACC contains floating point difference

ERROR:		?OVERFLOW ERROR if FACC too big.

EXAMPLE:	JSR $AF12	;SUBTRACT ARG FROM FACC, DIFF IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 7

NAME:		FADD	($AF18)

FUNCTION:	FACC = MEMORY + FACC

PREPARATION:	FACC contains floating point addend

.A = pointer (lsb) to packed floating point addend

.Y = pointer (msb) to packed floating point addend

SPECIAL NOTES:	The second addend *MUST* be in RAM-1 or common RAM in

packed format. FADD calls CONUPK to normalize it.

RESULT:		FACC contains floating point sum

ERROR:		?OVERFLOW ERROR if result too big

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* ADDEND

JSR $AF18	;ADD MEMORY TO FACC, SUM IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FADDT	($AF1B)

FUNCTION:	FACC = ARG + FACC

PREPARATION:	FACC contains floating point addend

ARG  contains floating point addend

ARISGN  ($70) contains EOR(FACSGN,ARGSGN)

.A   contains FACEXP

SPECIAL NOTES:	This routine is similar to FADD. The only

difference is the call to CONUPK.)

***********************************************

*  You  *MUST*  put resultant sign in ARISGN. *

*  You  *MUST*  load FACEXP ($63) immediately *

*  before call so that status flags are set!  *

***********************************************

RESULT:		FACC contains floating point sum

ERROR:		?OVERFLOW ERROR if result too big

EXAMPLE:	LDA FACSGN

EOR ARGSGN

STA ARISGN	;SET RESULTANT SIGN

LDA FACEXP	;SET STATUS FLAGS PER FACEXP

JSR $AF1B	;ADD ARG TO FACC, SUM IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 8

NAME:		FMULT	($AF1E)

FUNCTION:	FACC = MEMORY * FACC

PREPARATION:	FACC contains floating point multiplier

.A = pointer (lsb) to packed floating point multiplicand

.Y = pointer (msb) to packed floating point multiplicand

SPECIAL NOTES:	The multiplicand *MUST* be in RAM-1 or common RAM in

packed format. FMULT calls CONUPK to normalize it.

RESULT:		FACC contains floating point product

ERROR:		?OVERFLOW ERROR if result too big

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* MULTIPLICAND

JSR $AF1E	;MULTIPLY MEMORY BY FACC, PRODUCT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FMULTT	($AF21)

FUNCTION:	FACC = ARG * FACC

PREPARATION:	FACC contains floating point multiplier

ARG  contains floating point multiplicand

SPECIAL NOTES:	This routine is similar to FMULT. The only difference

is the call to CONUPK- FMULTT assumes you have already

loaded ARG with unpacked multiplicand.)

RESULT:		FACC contains floating point product

ERROR:		?OVERFLOW ERROR if result too big

EXAMPLE:	JSR $AF21	;MULTIPLY ARG BY FACC, PRODUCT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 9

NAME:		FDIV	($AF24)

FUNCTION:	FACC = MEMORY / FACC

PREPARATION:	FACC contains floating point divisor

.A = pointer (lsb) to packed floating point dividend

.Y = pointer (msb) to packed floating point dividend

SPECIAL NOTES:	The dividend *MUST* be in RAM-1 or common RAM in

packed format. FDIV calls CONUPK to normalize it.

RESULT:		FACC contains floating point quotient

ERROR:		?DIVISION BY ZERO ERROR if FACC zero

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* DIVIDEND

JSR $AF24	;DIVIDE MEMORY BY FACC, QUOTIENT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FDIVT	($AF27)

FUNCTION:	FACC = ARG / FACC

PREPARATION:	FACC contains floating point divisor

ARG  contains floating point dividend

ARISGN  ($70) contains EOR(FACSGN,ARGSGN)

.A   contains FACEXP

SPECIAL NOTES:	This routine is similar to FDIV. The only difference

is the call to CONUPK- FDIVT assumes you have already

loaded ARG with unpacked dividend.)

***********************************************

*  You  *MUST*  put resultant sign in ARISGN. *

*  You  *MUST*  load FACEXP ($63) immediately *

*  before call so that status flags are set!  *

***********************************************

RESULT:		FACC contains floating point quotient

ERROR:		?DIVISION BY ZERO ERROR if FACC zero

EXAMPLE:	LDA FACSGN

EOR ARGSGN

STA ARISGN	;SET RESULTANT SIGN

LDA FACEXP	;SET STATUS FLAGS PER FACEXP

JSR $AF27	;DIVIDE ARG BY FACC, QUOTIENT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 10

NAME:		LOG	($AF2A)

FUNCTION:	FACC = LOG(FACC)	natural logarithm (base e)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point logarithm

ERROR:		?ILLEGAL QUANTITY ERROR if FACC negative or zero

EXAMPLE:	JSR $AF2A	;FACC = LOG(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		INT	($AF2D)

FUNCTION:	FACC = INT(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point greatest integer

EXAMPLE:	JSR $AF2D	;FACC = INT(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		SQR	($AF30)

FUNCTION:	FACC = SQR(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point square root

ERROR:		?ILLEGAL QUANTITY ERROR if FACC negative

EXAMPLE:	JSR $AF30	;FACC = SQR(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		NEGOP	($AF33)

FUNCTION:	FACC = -FACC		(invert sign of FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point number with sign inverted

EXAMPLE:	JSR $AF33	;FACC = -FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 11

NAME:		FPWR	($AF36)

FUNCTION:	FACC = ARG ^ MEMORY

PREPARATION:	ARG contains floating point number

.A = pointer (lsb) to packed floating point power

.Y = pointer (msb) to packed floating point power

SPECIAL NOTES:	The power *MUST* be in ROM or common RAM in packed

format as FPWR calls MOVFM to unpack it into FACC.

RESULT:		FACC contains floating point result

ERROR:		?ILLEGAL QUANTITY ERROR if ARG negative

?OVERFLOW ERROR if result too big

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* POWER

JSR $AF36	;COMPUTE ARG ^ MEM, RESULT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		FPWRT	($AF39)

FUNCTION:	FACC = ARG ^ FACC

PREPARATION:	ARG  contains floating point number

FACC contains floating point power

.A   contains FACEXP

SPECIAL NOTES:	This routine is similar to FPWR. The only difference

is the call to MOVFM- FPWRT assumes you have already

loaded FACC with unpacked power.

***********************************************

*  You  *MUST*  load FACEXP ($63) immediately *

*  before call so that status flags are set!  *

***********************************************

RESULT:		FACC contains floating point result

ERROR:		?ILLEGAL QUANTITY ERROR if ARG negative

?OVERFLOW ERROR if result too big

EXAMPLE:	LDA FACEXP	;SET STATUS FLAGS PER FACEXP

JSR $AF39	;COMPUTE ARG ^ FACC, RESULT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		EXP	($AF3C)		(compute e ^ FACC)

FUNCTION:	FACC = EXP(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point result

ERROR:		?OVERFLOW ERROR if FACC too big

EXAMPLE:	JSR $AF3C	;FACC = EXP(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 12

NAME:		COS	($AF3F)

FUNCTION:	FACC = COS(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point cosine (in radians)

EXAMPLE:	JSR $AF3F	;FACC = COS(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		SIN	($AF42)

FUNCTION:	FACC = SIN(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point sine (in radians)

EXAMPLE:	JSR $AF42	;FACC = SIN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		TAN	($AF45)

FUNCTION:	FACC = TAN(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point tangent (in radians)

EXAMPLE:	JSR $AF45	;FACC = TAN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		ATN	($AF48)

FUNCTION:	FACC = ATN(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		FACC contains floating point arctangent (in radians)

EXAMPLE:	JSR $AF48	;FACC = ATN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 13

NAME:		ROUND	($AF4B)		(round to 40 bits of precision)

FUNCTION:	FACC = FACC + FACOV(msb)

PREPARATION:	FACC contains floating point number

FACOV (msb) contains 'extra' precision

RESULT:		none if FACC zero or FACOV (msb) zero

one extra bit ADDED to FACC lsb if FACOV (msb) is set

EXAMPLE:	JSR $AF4B	;ROUND FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		ABS	($AF4E)		(make FACSGN(msb) = $00)

FUNCTION:	FACC = ABS(FACC)

PREPARATION:	FACC contains (SIGNED) floating point number

RESULT:		FACC contains (POSITIVE) floating point

EXAMPLE:	JSR $AF4E	;FACC = ABS(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		SGN	($AF51)		(test SIGN of FACC)

FUNCTION:	.A = SGN(FACC)

PREPARATION:	FACC contains floating point number

RESULT:		.A  -->	$FF if FACC negative (FACC < 0)

$00 if FACC zero     (FACC = 0)

$01 if FACC positive (FACC > 0)

(status flags reflect contents of .A, carry invalid)

EXAMPLE:	JSR $AF51	;SGN(FACC)

; BEQ will trap =0

; BNE will trap <>0

; BMI will trap <0

; BPL will trap >=0	etc.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 14

NAME:		FCOMP	($AF54)		(compare FACC with MEMORY)

FUNCTION:	.A = FCOMP(FACC,MEMORY)

PREPARATION:	FACC contains floating point number

.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:	The number *MUST* be in ROM, or RAM currently in context

below ROM, in PACKED format. *** FACOV is significant!

RESULT:		.A  -->	$FF if FACC < MEMORY

$00 if FACC = MEMORY

$01 if FACC > MEMORY

(status flags reflect contents of .A, carry invalid)

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* NUMBER

JSR $AF54	;COMPARE FACC WITH MEMORY

; BEQ will trap FACC =  MEM

; BNE will trap FACC <> MEM

; BMI will trap FACC <  MEM

; BPL will trap FACC >= MEM	etc.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		RND0	($AF57)

FUNCTION:	FACC = random floating point number (0<n<1)

PREPARATION:	.A  -->	$00 to generate a 'true' random number

$01 to generate next random number in sequence

$FF to start a new sequence of random numbers

based upon current contents of FACC.

SPECIAL NOTES:	*MUST* be called with the system (BANK 15) in context.

*MUST* load .A immediately before call so that status

flags reflect contents of .A

RESULT:		FACC = floating point random number

EXAMPLE:	LDA #$FF	;START REPRODUCEABLE SEQUENCE BASED ON FACC

JSR $AF57

LDA #$01

JSR $AF57	;GENERATE (FIRST) RANDOM NUMBER IN SEQUENCE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 15

NAME:		CONUPK	($AF5A)

FUNCTION:	ARG = UNPACK(RAM_CONSTANT)

PREPARATION:	.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:	The number *MUST* be in RAM-1 or common RAM in packed format.

RESULT:		ARG	loaded with normalized floating point number

ARISGN	($6F) contains EOR(FACSGN,ARGSGN)

.A	contains FACEXP (status reflects contents of .A)

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* NUMBER IN RAM-1

JSR $AF5A	;LOAD ARG

; BEQ traps ARG = $00

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		ROMUPK	($AF5D)

FUNCTION:	ARG = UNPACK(ROM_CONSTANT)

PREPARATION:	.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:	The number *MUST* be in ROM or RAM currently in context

(otherwise identical to CONUPK).

RESULT:		ARG	loaded with normalized floating point number

ARISGN	($6F) contains EOR(FACSGN,ARGSGN)

.A	contains FACEXP (status reflects contents of .A)

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* NUMBER IN RAM-1

JSR $AF5D	;LOAD ARG

; BEQ traps ARG = $00

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		MOVFRM	($AF60)

FUNCTION:	FACC = UNPACK(RAM_CONSTANT)

PREPARATION:	.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:	The number *MUST* be in RAM-1 or common RAM in packed format.

MOVFRM is *NEW*, and not a true part of the math package!

***********************************************

*  *MUST*  be called from ROM or common RAM-  *

*  this routine RTS's with RAM-1 in context!  *

***********************************************

RESULT:		FACC	loaded with normalized floating point number

FACOV	($71) cleared

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* NUMBER IN RAM-1

JSR $AF60	;LOAD FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 16

NAME:		MOVFM	($AF63)

FUNCTION:	FACC = UNPACK(ROM_CONSTANT)

PREPARATION:	.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:	The number *MUST* be in ROM or RAM currently in context.

RESULT:		FACC	loaded with normalized floating point number

FACOV	($71) cleared

.A	contains FACEXP (status reflects contents of .A)

EXAMPLE:	LDA #<POINTER

LDY #>POINTER	;SET POINTER TO *PACKED* NUMBER IN ROM

JSR $AF63	;LOAD FACC

; BEQ traps ARG = $00

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		MOVMF	($AF66)

FUNCTION:	MEMORY = PACK(ROUND(FACC))

PREPARATION:	FACC contains floating point number

.X = pointer (lsb) to destination

.Y = pointer (msb) to destination

SPECIAL NOTES:	The destination will be to RAM currently in context

(*YOU* must configure memory- routine assumes writes

to ROM 'bleed through' to RAM in context, but be sure

I/O is turned off!)

RESULT:		FACC	will be ROUNDed and FACOV cleared.

MEMORY	thru MEMORY+4 will contain *PACKED* number.

EXAMPLE:	LDX #<POINTER

LDY #>POINTER	;POINTER TO RAM DESTINATION IN CURRENT BANK

JSR $AF66	;STORE FACC (PACKED)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

C128 BASIC 7.0 MATH PACKAGE  v0.2				Page 17

NAME:		MOVFA	($AF69)

FUNCTION:	FACC = ARG

PREPARATION:	ARG contains floating point number

RESULT:		FACC	contains same number as ARG

FACOV	($71) cleared

.A	contains FACEXP (but status invalid!)

EXAMPLE:	JSR $AF69	;COPY ARG TO FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NAME:		MOVAF	($AF6C)

FUNCTION:	ARG = FACC

PREPARATION:	FACC contains floating point number

RESULT:		FACC	will be ROUNDed and FACOV cleared.

ARG	contains same number as FACC

.A	contains FACEXP (but status invalid!)

EXAMPLE:	JSR $AF6C	;COPY FACC TO ARG

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

*** End of MATH ROUTINE documentation ***

REVISION HISTORY:

v0.1	-	4/22/86  Original limited distribution		(FAB)

v0.2	-	6/17/86  Corrections to: EXP   ($AF3C)		(FAB)

MOVMF ($AF66)