C128 OBJECT FILE LOADER

The loader is a program to load object files created by various

assemblers into memory.

Typically, object files are loaded using the loader, and then saved

using the rommed C128 machine code monitor.

The loader may be executed via the following BASIC statement:

BOOT "LOADER.BIN"

The loader resides in bank zero from $1300-$17FF. Attempts to

load files into this space will result in erratic system

operation.

If you desire to load object code into this space, it is first

neccesary to load the object into memory some place else, and

then use the machine language monitor to transfer the binary

image into its proper location before saving the file to disk.

The loader prompts the user for three pieces of information.

OBJECT FILE NAME:

This is the name of an object file in MOS Technology

format. This file may be ASCII or PETSCII in upper

or lower case.

OBJECT FILE DRIVE:

This is the unit number of the drive where the object

file resides.

LOAD ADDRESS:

Object files contain information about the address(es)

where data is supposed to be loaded into. Normally,

the loader loads object files into bank 0 , at the

address(es) specified by the object file.

If it is desired to load the information into another

region of memory, the the address of the first byte of

the region must be specified here. The format of

the load address is one to five hex digits in the

same format as accepted by the machine language monitor.

In function, the load address parameter is identical

to the OFFSET parameter found in other loaders. In use,

the load address parameter is easier to use because it

eliminates the need to calculate offsets, and allows

the user to think in terms of where he wants the data

to go.

During the load process, the loader displays the regions of

memory which the loader is writing information too.

Errors which occur during the load process are printed

on the screen. They are self explanatory.

MOS TECHNOLOGY OBJECT FILE FORMAT

The object file format used by all Commodore assemblers

and loaders is the MOS TECHNOLOGY format.

The MOS format consists of a series of DATA records followed

by an end of file ( EOF ) record. Each record consists of

a semicolon followed by a set of ASCII or PETSCII characters.

All characters in between records are to be ignored. Note that

a semicolon is the start of a record.

DATA records are organized as follows:

;NNAAAADDDDDDDDDDCCCC

Where

;	denotes the start of a record

NN	is a two byte hexadecimal representation

of the ( non-zero ) number of data bytes in

this record.

AAAA	is the four byte hexadecimal representation of

the sixteen bit address for the data in this record.

DD	Each DD is a two byte hexadecimal representation

of an eight bit value to loaded at the appropriate

address.

CCCC	represents the checksum of eight bit bytes in this

record.

EOF records are organized as follows:

;00RRRRCCCC

Where	;00	denotes this as an EOF record

RRRR	represents the total number of records in this

file.

CCCC	represents the checksum of the record count.