MAKEROM.DOC     CP/M 3.0 on the Commodore 128     28May87

To create the ROM BIOS used by  CP/M  on the  Commodore 128  follow the

procedure listed below.  You will need a 1571 or 1581 disk drive,  or a

1750 RAM disk, for this task.  If you do not have one of these devices,

you will have to make modifications to suit your system.  Slightly more

than a 200K space is needed for the procedure described.

1.  Create a work diskette by using the FORMAT  utility.  If  you  are

using a 1571 drive select the C128 double sided option. If you are

using a 1750 RAMdisk (drive M:), make sure it is empty.

2.  Copy the following files to your work disk, using the PIP utility:

From USER 0:   PIP.COM

MAC.COM

SUBMIT.COM

From USER 2:   CXROM.SUB

CXROM1.ASM

CXROM02.ASM

CXROM80.ASM

CXROMINT.ASM

CXROMK.ASM

CPM3.LIB

CXEQU.LIB

X6502.LIB

Z80.LIB

3.  To assemble new ROM code,  select  the  drive  which contains your

work disk (eg, A:)  and enter:  'SUBMIT CXROM'  and the  automatic

process will begin.  The output is in the file CXROM.HEX.

4.  This code should be regarded as an invariant portion of  CP/M  for

the Commodore 128,  and  is  available as source for informational

purposes only.  For alternatives  to  the  'ROM Loader',  see  the

source for the FORMAT utility (to support the 1581 disk drive, it

was necessary to place a new Loader on the 1581 system track).