PAGE 10                         C-64/128 KERMIT USER'S GUIDE

TROUBLESHOOTING

HERE ARE SOME COMMON PROBLEMS AND THEIR MOST LIKELY SOLU-

TIONS.  I HAVE FOUND THE BEST TROUBLESHOOTING COMES FROM

SOMEONE WHO HAS EXPERIENCE USING KERMIT, BUT THIS ISN'T

ALWAYS AVAILABLE.

1) PROBLEM: KERMIT WON'T BOOT.

SOLUTION: MAKE SURE ALL OF YOUR EQUIPMENT IS HOOKED UP

CORRECTLY.  IF THE RED ERROR LIGHT BLINKS, YOU MIGHT

HAVE A BAD DISK.  DR. EVIL LABS WILL REPLACE ANY

DEFECT KERMIT DISKS.

2) PROBLEM: I CAN'T GET OUT OF CONNECT MODE.

SOLUTION: REMEMBER TO TYPE <CTRL> - <Y>, LET UP, THEN

HIT THE LETTER "C".

3) PROBLEM: WHEN I CONNECT TO A HOST MACHINE, ALL I GET IS

GARBAGE.

SOLUTION: CHECK THE BAUD, PARITY AND WORD-SIZE PARAME-

TERS.  YOU MAY HAVE TO HANG UP AND TRY AGAIN IF YOU

CHANGE SOME OF THE PARAMETERS.

4) PROBLEM: I CAN'T SEE WHAT I AM TYPING, BUT I SEE WHAT

THE HOST MACHINE IS SENDING.

SOLUTION: YOU MIGHT NEED TO SET LOCAL-ECHO ON.

5) PROBLEM: I DOWNLOADED A BINARY FILE BUT THE FILE I

RECEIVED WAS GARBAGE.

SOLUTION: REMEMBER TO SET EIGHT-BIT QUOTING ON.  ALSO,

CHECK THE FILE TYPE ON THE HOST KERMIT.  IT SHOULD BE

SET TO BINARY AS WELL AS ON YOUR END.

6) PROBLEM: ALL OF A SUDDEN OUTPUT STOPPED APPEARING ON

THE SCREEN AND NOTHING I TYPE SHOWS UP EITHER.

SOLUTION: YOU MAY HAVE HIT THE <C=> KEY ACCIDENTLY,

WHICH PAUSES ALL I/O TEMPORARILY.  PRESSING IT AGAIN

WILL RE-START I/O.

C-64/128 KERMIT USER'S GUIDE                         PAGE 11

USING TERMINAL EMULATION

KERMIT IS A VERY POWERFUL TERMINAL EMULATOR.  IT CAN EMULATE

BOTH A VT-100 (WITH OR WITHOUT ADDED VT-102 FEATURES) AND A

VT-52.  EMULATION CAN BE TURNED COMPLETELY OFF AS WELL.

IT IS BEYOND THE SCOPE OF THIS MANUAL TO PRESENT EACH OF THE

ESCAPE SEQUENCES THAT A VT-52/100/102 TERMINAL USES.  THIS

SECTION WILL INSTEAD DESCRIBE IN GENERAL TERMS WHAT EACH OF

THE EMULATIONS CAN DO.

VT-100 EMULATION

MOST PEOPLE WILL USE KERMIT'S VT-100 EMULATION MORE THAN ANY

OTHER.  THE DEC VT-100 TERMINAL IS AN INDUSTRY STANDARD, AND

SO IS SUPPORTED ON VIRTUALLY EVERY MEDIUM-TO-LARGE-SIZED

OPERATING SYSTEM.  THE VT-100 TERMINAL PROVIDES YOU WITH

MANY POWERFUL FEATURES THAT MAKE COMMUNICATING WITH A REMOTE

HOST (ESPECIALLY AT SPEEDS UNDER 9600 BPS) MUCH MORE EFFI-

CIENT AND THEREFORE, QUICKER.

THE MAJOR VT-100 FEATURES ARE SUMMARIZED BELOW:

1) SCROLLING REGIONS.  THIS FEATURE OF THE VT-100 ALLOWS

SELECTED PARTS OF THE SCREEN TO BE SCROLLED, CUTTING

DOWN ON CHARACTER TRANSMISSION GREATLY-- MAKING

TELECOMMUNICATING FASTER.

2) TEXT ATTRIBUTES.  TEXT ON A VT-100 CAN BE DISPLAYED IN

NORMAL OR BOLD, AND BE VANILLA, UNDERLINED, OR BLINKING

(OR ANY LOGICAL COMBINATION), AND DISPLAYED ON EITHER A

DARK OR LIGHT BACKGROUND.  KERMIT HAS THE ADDED FEATURE

OF SELECTABLE COLORS FOR TEXT, BOLD TEXT, DARK-

BACKGROUND, LIGHT-BACKGROUND, AND BORDER FOR MOST

SCREEN DRIVERS.

3) NUMERIC KEYPAD.  A VT-100 HAS A NUMERIC KEYPAD THAT

INCLUDES FOUR PERSONAL FUNCTION (PF) KEYS.  KERMIT HAS

THIS FEATURE ON BOTH A C-64 & 128.

4) GRAPHICS CHARACTERS.  A VT-100 HAS A WIDE RANGE OF

GRAPHICS CHARACTERS THAT CAN BE USED FOR GRAPHS ON

CHARTS.

5) STATUS LINE.  KERMIT HAS AN EXTRA 25TH LINE THAT CAN BE

USED AS A SYSTEM STATUS LINE ON OPERATING SYSTEMS SUCH

AS UNIX.  NOTE THAT A STANDARD VT-100 TERMINAL DOES NOT

HAVE A SYSLINE.

FOR A CHART SHOWING WHAT KEYS HAVE BEEN RE-DEFINED IN KER-

MIT, PLEASE SEE APPENDIX C: KEYBOARD LAYOUTS.  ALSO, IF YOU

ARE USING KERMIT ON A MACHINE RUNNING UNIX, PLEASE SEE

PAGE 12                         C-64/128 KERMIT USER'S GUIDE

APPENDIX D: UNIX TERMCAP SETTINGS, FOR INFORMATION ON HOW TO

CUSTOMIZE YOUR TERMCAP TO TAKE FULL ADVANTAGE OF KERMIT'S

FEATURES.

NOT ALL OF THE VT-100'S FEATURES WERE FEASIBLE TO SUPPORT IN

KERMIT.  BELOW IS A LIST OF THE VT-100 FEATURES NOT SUP-

PORTED:

1) NO 132 COLUMN MODE

2) NO DOUBLE HIGH/DOUBLE WIDE CHARACTERS

3) LIMITED TERMINAL REPORTS (ONLY THE IMPORTANT ONES ARE

EMULATED)

4) NO FOREIGN CHARACTER SETS

5) NO SMOOTH SCROLLING

6) NO TERMINAL LEDS

FOR RELATED INFORMATION, PLEASE SEE THE SCREEN DRIVER COM-

PARISON SECTION.

VT-102 EMULATION

ONLY THE FOUR MOST IMPORTANT VT-102 TERMINAL FEATURES ARE

PRESENT IN KERMIT.  THESE ARE INSERT CHARACTER, DELETE CHAR-

ACTER, INSERT LINE, AND DELETE LINE.  THE INSERT CHARACTER

FUNCTION IS REALLY A SWITCHABLE INSERT/REPLACE MODE.  NO

OTHER VT-102 FEATURE ARE SUPPORTED.

VT-52 EMULATION

KERMIT EMULATES A VT-52 TERMINAL AS A SUBSET OF ITS VT-100

EMULATION, JUST AS A REAL VT-100 TERMINAL DOES.  IT IS A

COMPLETE EMULATION, EXCEPT THAT THE GRAPHICS CHARACTER SET

IS THE VT-100'S, NOT A TRUE VT-52'S.  ALL OF THE REAL VT-100

TERMINALS WE HAVE SEEN ALSO USE THE VT-100 CHARACTER SET IN

PLACE OF THE TRUE VT-52 SET IN VT-52 MODE.

C-64/128 KERMIT USER'S GUIDE                         PAGE 13

TEKTRONIX 4010 EMULATION

KERMIT SUPPORTS A LIMITED KIND OF TEKTRONIX 4010 EMULATION.

TWO RESOLUTIONS ARE SUPPORTED: 320 X 200 (C-64) AND 640 X

200 (C-128 80 COL.).  YOU CAN USE THE TEKTRONIX EMULATION

EVEN IF YOU ARE USING A BI-80 CARD, AS KERMIT WILL AUTOMATI-

CALLY SWITCH BETWEEN THE APPROPRIATE GRAPHICS SCREEN AND THE

BI-80.  ALL PLOTS AND TEXT ARE PLOTTED USING THE CURRENT

CHARACTER COLOR AND ARE PLOTTED ON THE CURRENT BACKGROUND

COLOR.  BOTH THE 320 X 200 AND 640 X 200 SCREENS ARE A LIT-

TLE TOO WIDE HORIZONTALLY SCALE-WISE COMPARED WITH A REAL

TEKTRONIX 4010 TERMINAL.  WE THOUGHT IT WAS BETTER TO USE

THE FULL WIDTH OF THE SCREEN GIVEN THE LIMITED RESOLUTION

THAN TO HAVE PERFECT SCALING.

KERMIT SUPPORTS THREE TEK MODES: ALPHA TEXT, POINT PLOT, AND

VECTOR PLOT, FOR A TOTAL OF EIGHT DIFFERENT TEK COMMANDS.

THE COMMANDS ARE:

SEQUENCE                ACTION

<ESC> FF (FORM FEED)    ACTIVATE TEK EMULATION AND CLEAR SCREEN

RS (RECORD SEPARATOR)   SWITCH TO INCREMENTAL PLOT MODE

US (UNIT SEPARATOR)     SWITCH TO ALPHA TEXT MODE

GS (GRAPHICS START)     SWITCH TO VECTOR PLOT MODE

CAN (CANCEL)            DEACTIVATE TEK EMULATION AND CLEAR SCREEN

TEK MODE IS ALSO EXITED WHEN ANY VT-100/52 ESCAPE SEQUENCE

IS RECEIVED.  THIS INCLUDES ANY SEQUENCES BEGINNING WITH

<ESC> "[" OR <ESC> <UPPER-CASE LETTER>.  ALSO, ONCE TEK MODE

IS ACTIVATED, ANY FURTHER <ESC> FF SEQUENCES RECEIVED WILL

CLEAR THE CURRENT GRAPHICS SCREEN.  FINALLY, THE CURRENT

TEXT SCREEN IS NOT SAVED WHEN TEK MODE IS ENTERED, AND THE

CURRENT TEK SCREEN IS NOT SAVED WHEN TEK MODE IS EXITED.

ALPHA TEXT MODE

ALPHA TEXT MODE USES THE STANDARD ASCII CHARACTER SET AND

CAN DISPLAY 79 COLUMNS BY 24 LINES OF TEXT IF TEK MODE IS

ENTERED FROM ANY OF THE SCREEN DRIVERS EXCEPT C-64 40

COLUMNS.  IF TEK MODE IS ENTERED FROM THE C-64 40 COLUMN

SCREEN DRIVER, UP TO 40 COLUMNS OF TEXT CAN BE DISPLAYED.

THIS MODE IS USED GENERALLY TO LABEL GRAPHS.  NO ON-SCREEN

EDITING SHOULD BE ATTEMPTED WHILE IN THIS MODE.

POINT PLOT MODE

IN THIS MODE THE DRAWING PEN IS GIVEN A DIRECTION TO MOVE

AND WHETHER TO HAVE THE PEN UP (NO LINE DRAWN) OR DOWN (LINE

PAGE 14                         C-64/128 KERMIT USER'S GUIDE

DRAWN) WHEN MOVING.  THIS ALLOWS CONSIDERABLE FREEDOM IN

DRAWING ODD-SHAPED OBJECTS.

VECTOR PLOT MODE

IN VECTOR PLOT MODE, THE ENDPOINTS OF A LINE TO BE DRAWN ARE

SPECIFIED.  THIS ALLOWS LINES TO BE DRAWN QUICKLY.

C-64/128 KERMIT USER'S GUIDE                         PAGE 15

SCREEN DRIVER COMPARISON

EACH OF KERMIT'S SCREEN DRIVERS HAS DIFFERENT CAPABILITIES.

BELOW IS A DISCUSSION OF THE VARIOUS LIMITATIONS OF THE DIF-

FERENT SCREEN DRIVERS.

C-128 80 COLUMN DRIVER

THIS DRIVER IS THE BEST THAT KERMIT HAS.  ALL VIDEO ATTRI-

BUTES ARE SUPPORTED WITH NO LIMITATIONS, AND THE HIGHEST TEK

RESOLUTION (640 X 200) IS AVAILABLE. THE ONLY FEATURE NOT

AVAILABLE WITH THIS DRIVER, THAT IS ON SOME OTHERS, IS A

BORDER.  THIS IS BECAUSE THE C-128'S 80 COLUMN CHIP (THE

8563 VDC) DOES NOT SUPPORT A BORDER.

ONE NOTE: MANY PEOPLE WHO USE THE 80-COLUMN MODE ON THE C-

128 DO SO ON A MONOCHROME MONITOR.  WITH THIS SETUP, ONLY

THREE COLORS ARE POSSIBLE: BRIGHT, NORMAL, AND BLACK.  THIS

CAN POSE A PROBLEM WHEN TRYING TO SET UP READABLE LIGHT- AND

DARK-BACKGROUND SCREENS.  ONE SOLUTION TO THIS IS TO BUY A

GREY SCALE ATTACHMENT FOR THE 80-COLUMN VIDEO CABLE, WHICH

WILL CREATE MORE SHADES OF INTENSITY.

C-64 80 COLUMN DRIVER

THIS SCREEN DRIVER IS TRADITIONALLY THE MOST POPULAR, MAINLY

BECAUSE MORE PEOPLE HAVE C-64'S THAN C-128'S.  WHEN THIS

DRIVER IS IN USE, EVEN ON A C-128, THE TEK 320 X 200 MODE

WILL BE USED IF GRAPHICS MODE IS ACTIVATED. EVERY FEATURE IS

AVAILABLE WITH THIS DRIVER, BUT THERE IS ONE MINOR LIMITA-

TION.

BECAUSE OF THE WAY HI-RES GRAPHICS SCREENS ARE MAPPED ON A

C-64, IF TWO CHARACTERS WITH DIFFERENT EITHER BOLD OR FLASH-

ING ATTRIBUTES ARE PLACED ADJACENT HORIZONTALLY, THE FIRST

CHARACTER WILL NOT HAVE THE CORRECT ATTRIBUTE.  FOR EXAMPLE,

IF A BOLD CHARACTER IS PRINTED IMMEDIATELY FOLLOWED BY AN

FLASHING CHARACTER, THE BOLD CHARACTER WILL CHANGE TO A

FLASHING CHARACTER.  THIS CAN EASILY BE AVOIDED BY INSERTING

A SPACE BETWEEN THE TWO CHARACTERS.

BATTERIES INCLUDED BI-80 DRIVER (STANDARD)

THE BI-80, AN EXCELLENT 80-COLUMN CARD FOR THE C-64 (ALSO

C-128 IN 64 MODE), IS NO LONGER IN PRODUCTION AND GENERALLY

VERY HARD TO FIND USED.  WE ADDED SUPPORT FOR IT BECAUSE IT

OFFERS THE SAME TEXT RESOLUTION AS THE C-128'S 80 COLUMN

SCREEN, MAKING IT CLEARER THAN THE STANDARD C-64 80 COLUMN

SCREEN.

PAGE 16                         C-64/128 KERMIT USER'S GUIDE

THERE ARE SEVERAL LIMITATIONS WHEN USING THIS DRIVER.  THE

MAIN LIMITATION IS THAT THE BI-80 CHARACTER SET IS NOT REDE-

FINABLE THROUGH SOFTWARE, MEANING THAT SOME CHARACTERS LIKE

THE BRACES, TILDE, UNDERSCORE, ETC., ARE NOT AVAILABLE.

ALSO, THE VT-100/52 GRAPHICS CHARACTER SET IS NOT AVAILABLE.

A CLOSEST MATCH WAS MADE ON MANY OF THE CHARACTERS, BUT MANY

OF THEM LOOK PRETTY UNRECOGNIZABLE.

ALSO, THE ONLY CHARACTER ATTRIBUTE AVAILABLE IS REVERSE.

ALL OTHER MODES, SUCH AS BLINKING, BOLD, AND UNDERLINED, ARE

MAPPED INTO REVERSE, SO YOU CAN AT LEAST TELL WHEN THE TEXT

IS NOT PLAIN VANILLA.

FINALLY, NO COLOR CHANGE OPTIONS ARE AVAILABLE AS THE BI-80

CARD OPERATES ONLY IN BLACK AND WHITE MODE WITH NO BORDER.

BATTERIES INCLUDED BI-80 DRIVER (CUSTOM)

THIS DRIVER IS FOR USE WITH THE CUSTOM BI-80 CHARACTER ROM

AVAILABLE FROM DR.  EVIL LABORATORIES.  THIS DRIVER MAKES

USE OF THE RE-DEFINED CHARACTERS IN THE REPLACEMENT ROM.

THE FULL ASCII CHARACTER SET IS AVAILABLE, AS WELL AS THE

VT-100/52 GRAPHICS CHARACTER SET.  ALL OTHER RESTRICTIONS

FROM THE STANDARD BI-80 DRIVER APPLY.

PLEASE SEE APPENDIX E: BI-80 REPLACEMENT ROM INFORMATION FOR

DETAILS ON HOW TO OBTAIN A REPLACEMENT CHARACTER ROM.

C-64 40-COLUMN DRIVER

THIS DRIVER SUPPORTS ALL CHARACTER ATTRIBUTES AND SCREEN

FUNCTIONS.  ITS MAIN LIMITATION IS ITS WIDTH-- 40 COLUMNS IS

GENERALLY NOT VERY USEFUL WHEN COMMUNICATING WITH MOST LARGE

COMPUTER SYSTEMS.