COMMODORE 64/128 KERMIT USER'S GUIDE

VERSION 2.1

PRELIMINARY DOCUMENTATION -- SUBJECT TO REVISION

VERSION 2.0 & 2.1 ENHANCEMENTS BY RAY MOODY

USER'S GUIDE BY KENT SULLIVAN

DR. EVIL LABORATORIES

P. O. BOX 190

ST. PAUL, IN 47272

THIS PRELIMINARY DOCUMENT WAS PRODUCED ON: APRIL 18, 1988

I

SUMMARY OF C-64/128 KERMIT VERSION 2.1

WHAT KERMIT IS

THE TERM "KERMIT" DENOTES BOTH A SPECIFIC PROTOCOL FOR

TRANSFERRING FILES BETWEEN COMPUTERS AND THOSE COMPUTER PRO-

GRAMS WHICH CAN DO THE TRANSFERRING.  KERMIT AS A PROTOCOL

IS OFTEN CALLED THE KERMIT PROTOCOL TO DISTINGUISH IT FROM

AN ACTUAL COMPUTER PROGRAM.  VARIOUS VERSIONS OF THE KERMIT

PROGRAM FOR DIFFERENT COMPUTERS ALSO HAVE LONGER, OFFICIAL

NAMES TO DISTINGUISH THEMSELVES.  KERMIT FOR THE COMMODORE

64/128 WAS ORIGINALLY KNOWN AS KERMIT-65.  C-64/128 KERMIT

IS A PUBLIC-DOMAIN TELECOMMUNICATIONS PROGRAM THAT UTILIZES

THE KERMIT PROTOCOL FOR FILE TRANSFERS.

THE "65" IN KERMIT-65 SIGNIFIES THAT THE COMMODORE 64 HAS AS

ITS MICROPROCESSOR THE 6510A, A MEMBER OF THE 6500 SERIES

FAMILY, WHICH ALSO INCLUDES THE APPLE II.  KERMIT FOR THE

COMMODORE 64 WAS ORIGINALLY TRANSLATED FROM APPLE II KERMIT.

KERMIT HAS BEEN IN EXISTENCE FOR THE C-64 (AS THE COMMODORE

64 IS ALSO KNOWN) SINCE MARCH, 1984.

HALL OF FAME

MANY PEOPLE HAVE WORKED ON KERMIT, CONTINUALLY ADDING

IMPROVEMENTS AND FIXING BUGS.  BELOW IS A LIST OF THE PEOPLE

WHO HAVE WORKED ON THE COMMODORE 64/128 KERMIT PROJECT OVER

THE YEARS.

PROGRAMMING:

DAVID DERMOTT

ERIC LAVITSKY

MICHAEL MARCHIONDO

RAY MOODY

FRANK PRINDLE

DOCUMENTATION:

ERIC LAVITSKY

KENT SULLIVAN

VERSION 2.1 IMPROVEMENTS

THIS SECTION DESCRIBES THE IMPROVEMENTS THIS VERSION OF KER-

MIT HAS OVER THE PREVIOUS VERSION (2.0).  IF YOU ARE A

FIRST-TIME USER OF KERMIT, YOU MAY WANT TO MOVE DIRECTLY TO

THE INTRODUCTION TO THE KERMIT USER'S GUIDE SECTION.

II

VERSION 2.1 IS THE SECOND VERSION OF KERMIT RAY AND I HAVE

WORKED ON.  THE MAIN IMPROVEMENTS TO VERSION 2.1 ARE AS FOL-

LOWS:

+ ENHANCED DEC VT-100 TERMINAL EMULATION WITH SUPPORT FOR

THE VT-100 KEYPAD.  KERMIT SHOULD NOW WORK FINE ON

OPERATING SYSTEMS SUCH AS VMS.  V2.1 HAS SOME VT-102

FEATURES ADDED AS WELL: INSERT/DELETE LINE/CHARACTER.

+ LIMITED TEKTRONIX 4010 GRAPHICS TERMINAL EMULATION.  KER-

MIT CAN PLOT BOTH GRAPHICS AND TEXT WITH A RESOLUTION OF

320 X 200 (C-64) OR 640 X 200 (C-128).

+ COMMODORE 128 GREY KEY AND NUMERIC KEYPAD SUPPORT.  THE

GREY KEYS AND THE NUMERIC KEYPAD, NOT NORMALLY ACCESSIBLE

ON A C-128 IN C-64 MODE, ARE ACTIVE WITHIN KERMIT.

+ A SPECIAL FILE-TYPE FOR TRANSFERRING C POWER (NOW POWER

C) SOURCE CODE FILES.  YOU CAN NOW DOWNLOAD C SOURCE CODE

ASCII FILES AS WELL AS UPLOAD C POWER SOURCE FILES.  ALL

NECESSARY CHARACTER TRANSLATIONS ARE HANDLED AUTOMATI-

CALLY.

+ A SCREEN DRIVER FOR THE BATTERIES INCLUDED BI-80 80

COLUMN CARD.  THIS DEVICE GIVES A DISPLAY AS NICE AS THE

COMMODORE 128'S 80-COLUMN HARDWARE, BUT LACKS MANY ATTRI-

BUTES SUCH AS BLINKING AND BOLD.

IN ADDITION TO THESE MAJOR IMPROVEMENTS, MANY SMALLER

CHANGES WERE MADE.  SEVERAL BUGS WERE SQUASHED THAT AFFECTED

VT-100 EMULATION, THE STATUS COMMAND, AND OTHER THINGS.  FOR

RELATED INFORMATION, PLEASE SEE APPENDIX A: IMPLEMENTATION

NOTES.

III

TABLE OF CONTENTS

SUMMARY OF C-64/128 KERMIT VERSION 2.1 ................    I

WHAT KERMIT IS ......................................    I

HALL OF FAME ........................................    I

VERSION 2.1 IMPROVEMENTS ............................    I

INTRODUCTION TO THE KERMIT USER'S GUIDE ...............    1

WHAT YOU NEED TO KNOW ...............................    1

RECOMMENDED INTRODUCTORY & SUPPLEMENTARY TEXTS ......    1

DR. EVIL LABS' COMMITMENT TO THE KERMIT PROJECT .....    2

SYNTAX OF COMMANDS ..................................    2

INTRODUCTORY TUTORIAL .................................    3

BOOTING KERMIT ......................................    3

CHOOSING A SCREEN DRIVER ............................    3

THE THREE MODES OF KERMIT ...........................    4

COMMAND MODE ......................................    4

CONNECT MODE ......................................    6

FILE-TRANSFER MODE ................................    7

TROUBLESHOOTING .......................................   10

USING TERMINAL EMULATION ..............................   11

VT-100 EMULATION ....................................   11

VT-102 EMULATION ....................................   12

VT-52 EMULATION .....................................   12

TEKTRONIX 4010 EMULATION ..............................   13

ALPHA TEXT MODE .....................................   13

POINT PLOT MODE .....................................   13

VECTOR PLOT MODE ....................................   14

SCREEN DRIVER COMPARISON ..............................   15

C-128 80 COLUMN DRIVER ..............................   15

C-64 80 COLUMN DRIVER ...............................   15

BATTERIES INCLUDED BI-80 DRIVER (STANDARD) ..........   15

BATTERIES INCLUDED BI-80 DRIVER (CUSTOM) ............   16

C-64 40-COLUMN DRIVER ...............................   16

TRANSFERRING FILES ....................................   17

CHOOSING THE CORRECT FILE-TYPE ......................   17

ASCII FILE-TYPE ...................................   17

BINARY FILE-TYPE ..................................   17

C-POWER FILE-TYPE .................................   18

PETSCII FILE-TYPE .................................   18

SCRIPT FILE-TYPE ..................................   18

SETTING THE FILE-BYTE-SIZE ..........................   18

USING EIGHT-BIT-QUOTING .............................   19

SPECIAL FILE-TRANSFER MODE FEATURES .................   19

COMMAND REFERENCE .....................................   20

COMMAND LIST ........................................   20

COMMAND SUMMARY .....................................   20

BYE ...............................................   20

CONNECT ...........................................   20

DIRECTORY .........................................   21

DISK ..............................................   21

EXIT ..............................................   22

FINISH ............................................   22

IV

GET ...............................................   22

QUIT ..............................................   22

RECEIVE ...........................................   23

RESTORE ...........................................   23

SAVE ..............................................   23

SEND ..............................................   24

SET ...............................................   24

SET BAUD ........................................   24

SET BOLD ........................................   25

GENERAL NOTES ON COLOR ........................   25

SET BORDER ......................................   25

SET CHARACTER ...................................   25

SET DARK-BACKGROUND .............................   26

SET DEBUGGING ...................................   26

SET EIGHT-BIT-QUOTING ...........................   26

SET ESCAPE ......................................   26

SET FILE-BYTE-SIZE ..............................   27

SET FILE-TYPE ...................................   27

SET FILE-WARNING ................................   27

SET FLOW-CONTROL ................................   28

SET IBM .........................................   28

SET LIGHT-BACKGROUND ............................   28

SET LOCAL-ECHO ..................................   29

SET PARITY ......................................   29

SET RECEIVE .....................................   29

SET RECEIVE EIGHT-BIT-QUOTE-CHAR ..............   30

SET RECEIVE END-OF-LINE .......................   30

SET RECEIVE PACKET-LENGTH .....................   30

SET RECEIVE PAD-CHAR ..........................   31

SET RECEIVE PADDING ...........................   31

SET RECEIVE QUOTE-CHAR ........................   31

SET RECEIVE TIMEOUT ...........................   31

SET RS232-REGISTERS .............................   32

SET SCREEN-DRIVER ...............................   32

SET SEND ........................................   32

SET TERMINAL-EMULATION ..........................   33

SET WORD-SIZE ...................................   33

SHOW ..............................................   33

STATUS ............................................   34

APPENDIX A: IMPLEMENTATION NOTES ......................   35

SUMMARY OF KERMIT CAPABILITIES ......................   35

KNOWN RESTRICTIONS ..................................   35

FUTURE PLANS FOR KERMIT .............................   36

APPENDIX B: DEFAULT INITIALIZATION SETTINGS ...........   37

APPENDIX C: KEYBOARD LAYOUTS ..........................   38

COMMODORE 64 MAP ....................................   38

COMMODORE 128 MAP ...................................   39

APPENDIX D: UNIX TERMCAP SETTINGS .....................   40

APPENDIX E: BI-80 REPLACEMENT ROM INFORMATION .........   41

C-64/128 KERMIT USER'S GUIDE                          PAGE 1

INTRODUCTION TO THE KERMIT USER'S GUIDE

WHAT YOU NEED TO KNOW

THIS MANUAL WILL INTRODUCE KERMIT TO NEW USERS AS WELL AS

PROVIDE AN ORGANIZED REFERENCE FOR EXPERIENCED USERS.  SOME

BASIC KNOWLEDGE OF COMPUTERS IS REQUIRED TO USE KERMIT.  NEW

USERS SHOULD UNDERSTAND THE BASICS OF COMPUTERS AND TELECOM-

MUNICATIONS.  PLEASE SEE RECOMMENDED INTRODUCTORY & SUPPLE-

MENTARY TEXTS BELOW FOR A SHORT LIST OF BOOKS RECOMMENDED

FOR BACKGROUND KNOWLEDGE.

DON'T BECOME FRUSTRATED IF YOU HAVE PROBLEMS INITIALLY USING

KERMIT.  CAREFUL READING OF THIS MANUAL WILL CLEAR UP MOST

PROBLEMS THAT YOU MIGHT HAVE.  IF YOU HAVE QUESTIONS, DON'T

BE AFRAID TO ASK AROUND.  KERMIT IS A VERY POPULAR PROGRAM,

SO MANY PEOPLE KNOW HOW TO USE IT.  YOU MAY DIRECT ANY QUES-

TIONS TO RAY OR ME AT THE ADDRESS ON THE FRONT OF THIS

MANUAL AS WELL.

THIS MANUAL IS A PRELIMINARY VERSION AND IS SUBJECT TO REVI-

SION.  IT WILL BE AVAILABLE IN FINAL PRINTED FORM (LASER-

PRINTED!) BEGINNING IN MAY OR JUNE 1988.  THE FINAL VERSION

WILL ALSO BE ON-DISK AS THIS VERSION IS.  THIS MANUAL CAN BE

PRINTED OUT USING MOST ANY SEQUENTIAL FILE PRINTER OR WORD

PROCESSOR.  ALL SPECIAL FEATURES SUCH AS UNDERLINING AND

BOLD HAVE BEEN REMOVED FROM THE ON-DISK MANUAL SO IT SHOULD

BE PRINTABLE ON ANY PRINTER.

RECOMMENDED INTRODUCTORY & SUPPLEMENTARY TEXTS

THE FOLLOWING IS A SHORT LIST OF RECOMMENDED BACKGROUND

TEXTS.  THE AREA COVERED IS LISTED FIRST, FOLLOWED BY THE

TITLE OF THE TEXT AND PUBLISHING INFORMATION.

+ BASIC COMMODORE COMPUTING: COMMODORE 64/64C/128 USER'S

GUIDES.

+ BASIC TELECOMMUNICATIONS: COMPUTE!'S TELECOMPUTING ON

THE COMMODORE 64, COMPUTE! BOOKS.

+ THE KERMIT PROTOCOL: KERMIT: A FILE-TRANSFER PROTOCOL

FOR UNIVERSITIES, 2 PARTS, BYTE MAGAZINE, JUNE & JULY,

1984.

PAGE 2                          C-64/128 KERMIT USER'S GUIDE

DR. EVIL LABS' COMMITMENT TO THE KERMIT PROJECT

ALL KERMIT DISTRIBUTION FOR VERSIONS THAT RAY AND I HAVE

WORKED ON (VERSIONS 2.0 AND LATER) IS HANDLED BY DR. EVIL

LABORATORIES.  DR. EVIL LABORATORIES IS A SMALL SOFTWARE

COMPANY IN WHICH RAY AND I ARE PARTNERS.  OUR COMPANY HAS

THE FACILITIES TO DISTRIBUTE KERMIT MUCH MORE EFFICIENTLY

THAN EITHER OF US COULD PERSONALLY.  ALSO, DR. EVIL LABS HAS

A PERMANENT ADDRESS, SOMETHING THAT WE, BEING IN COLLEGE,

DON'T HAVE.  IF YOU HAVE ANY QUESTIONS OR RECOMMENDATIONS

CONCERNING KERMIT, PLEASE CONTACT US AT DR. EVIL LABS.

SYNTAX OF COMMANDS

MOST KERMIT COMMANDS ARE SIMPLE ENGLISH PHRASES.  COMMANDS

ARE REPRESENTED IN THIS DOCUMENT IN CAPITAL LETTERS (I.E.

CONNECT) TO SET THEM OFF FROM ANY SURROUNDING TEXT.  MANY

STANDARD ABBREVIATIONS FOR KEYS ARE USED.  BELOW IS A TABLE

SHOWING THE ABBREVIATIONS AND WHAT THEY REPRESENT:

ABBREVIATION       ACTUAL KEY

<C=>               COMMODORE

<CLR>              CLEAR (SHIFTED HOME)

<CTRL>             CONTROL

<DEL>              DELETE

<INST>             INSERT (SHIFTED DELETE)

<SPACE>            SPACEBAR

<RETURN>           RETURN

<F1>,<F2>...<F8>   FUNCTION KEY 1,2...8

FOR A LIST OF KEYS THAT HAVE BEEN RE-DEFINED TO IMPLEMENT

SPECIAL CHARACTERS, PLEASE SEE APPENDIX C: KEYBOARD LAYOUTS.