C128 ROM RELEASE NOTES:		318020-06, 318023-03	      05/30/88

THE FOLLOWING MODIFICATIONS HAVE BEEN MADE TO THE 318020-05 KERNEL  CODE TO

CREATE A NEW ROM SET RELEASED ON 05/30/88.  THIS  RELEASE IS FOR USE IN THE

FOLLOWING SYSTEMS:  C128, C128D, C128CR, C128DCR.  ROMS WILL BE BUILT FOR

ONLY THE CURRENT 32K BYTE "CR" SYSTEMS, HOWEVER (318023-03).

1.  # 318023-03 --> EDITOR, KERNEL, CP/M ($C000-$FFFF, U32) cksum= E424

2.  # 318020-06 --> EDITOR, KERNEL, CP/M ($C000-$FFFF)      cksum= DFC4

1.  SECURE.  This kernel initialization routine has been modified

to accomodate technological advances in DRAMs.  The discharge

rates for data cells has lengthened considerably and it is no

longer possible to rely upon the  loss  of data as indicative

of a  power cycle as opposed to a  reset.  The SECURE routine

has been  patched  to  examine  the contents of the  8568 VDC

registers  which  will  contain  $FF if power cycled but will

retain their previously  programmed data through resets.  The

existing detection of a "CBM" RAM key in bank one remains for

compatability.  This patch required new code in patch area.

2.  DLABYE.  This kernel  serial  I/O routine has been patched to

provide a longer delay.  This will eliminate certain randomly

occurring DEVICE NOT READY errors, particularly at 2MHz. This

was accomplished via single byte inline patch.

3.  OPN232.  This kernel  routine  has  been patched to correctly

return error status.  When  implementing  an X-line interface

it was possible to receive an error when there was none. This

was accomplished via single byte inline patch.

4.  The ROM  signature at location $CFFC  and  $CFFD  (lo/hi)  is

$8C3E.

5.  The ROM revision byte at location $CFFE, has incremented from

$01 to $02.

6.  The ROM checksum byte at  location  $CFFF,  has  changed from

$3C to $3B.

7.  The Kernel revision byte at location  $FF80  has  incremented

from $01 to $02.

Fred Bowen        05/30/88