Game Console				Fred Bowen	21 June 1990

The following text describes the changes I made to the C64C ROM code

(CBM part number 251913-01) for the Game Console.  The changes were

necessary to prevent the normal C64 title screen from appearing when

the Console is turned on without a game cartridge installed.

1. BASIC code (LOROM) is unchanged except for the initialization of

the indirect vector IMAIN, which now points to the Console powerup

display generator located in the Kernel (HIROM).

2. The cassette drivers have been completely removed from the Kernel

code (HIROM) and replaced by the Console title sceen generator and

associated data.  Those I/O handlers (such as BASIN, BSOUT, LOAD

and SAVE) which call the cassette drivers have been patched such

that all attempts to communicate with a cassette fail with either

a file_not_found or a stop_key_detected error.  Because some C64

software utilized specific cassette code (such as CMPSTE, WRTBK,

TNIF, STKY, and INCSAL), such ROM entries have been preserved for

compatibility.  All other I/O drivers (such as serial bus and

RS232 drivers) are completely intact and functional.

3. The Kernel RAMTAS routine has been patched to allow the Console to

complete system initialization in a fraction of the time the C64

requires.  RAM contents and system initialization is otherwise

identical to that of a C64.

The Console powerup screen generator was coded such that application

software could utilize it as a subroutine to draw an untitled window

with a dropshadow.  It could be used, for example, to present a common

menu display when coupled with the application's custom prompting and

selection routines.  Such a menu/selection interface was utilized by

the C64 Arcade game cartridge.  It should be noted, however, that

applications that use code unique to the Game Console will not work

correctly when applied to a C64.  The call address is $F72C, and the

routine uses 3 bytes of base page memory at $FB-$FD.

Cartridges such as Commodore's Super Expander 64 will work in the

Game Console, although the usefulness of such BASIC applications is

questionable in a system with a keyboard or I/O ports.  It does

illustrate, however, that cartridge applications written in BASIC

and downloaded into RAM from the cartridge will work.  All that is

necessary is for the cartridge to install the usual IMAIN vector

($302 <-- $A483).

Game Console applications must avoid specific ROM calls, avoid the use

of undefined 6502 opcodes, and avoid reserved I/O locations to remain

compatible with the proposed "C65" system.  More information concerning

the "C65" system will be available at a later time.