The cart has a four bit latch sitting at $df00.(IO2).

Reset clears the latch to all zeros.

Writing to DF00 has the following effects on a per bit basis:

D0:	Goes to A13 on the roms.

D1:	Goes to A14 on the roms.

D2:	Goes to XROM and GAME.

( writing a one here sets xrom and game,

write a zero here pulls ( asserts ) xrom and game.

Reset, clears this, therefore asserting xrom and game.

D3:	Writing a 1 here causes the latch to be disabled

from any further writes until RESET. Thus, whatever

you gots on D0-D2 when you write a one here is latched

forever.

As for the roms:

ROML and ROMH enable one of the two ROMS for output

( just like before ).

A13 and A14 come from the latch allowing one of four rom areas

to selected as active.

Thus the rom images might look something like:

ADDRESS			ROM0			ROM1

-----------	-----------------	---------------------

$0000-$1FFF	ROM0 Jack Attack	ROM1 Jack attack.

$800 bytes boot code.

$2000-$3FFF	ROM0 Soccer		ROM1 Soccer

$4000-$5FFF	Chess image $0000-$1fff	Chess image $2000-$3fff

$6000-$7FFF	Chess image $4000-$5fff	Chess image $6000-$7fff

Note that it is impossible to make the ROMS entirely disappear

from the C128 memory map. However, they will go away from the

C64 map if GAME and XROM are taken high ( 1 written to B2 at

$df00 ).

There is a small black jumper on the cartridge externder board.

Reomving this will disable the carts ability to assert

XROM and GAME. This is kind of convenient if you want to look

at the roms using your 128.