ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 1

'declare 06/05/84'   DECLARE.SRC

Error Addr  Code          Seq   Source statement

1   	.nam ted_kernal

2

3   	.include   disclaim

4   ;  ******************************************************************

5   ;  *                                                                *

6   ;  *     PPPPPPPP   LLL      UUU   UUU   SSSSSSS           444      *

7   ;  *     PPP   PPP  LLL      UUU   UUU  SSS   SSS        44444      *

8   ;  *     PPP   PPP  LLL      UUU   UUU  SSS             44 444      *

9   ;  *     PPPPPPPP   LLL      UUU   UUU   SSSSSSS      44   444      *

10   ;  *     PPP        LLL      UUU   UUU        SSS     444444444     *

11   ;  *     PPP        LLL      UUU   UUU  SSS   SSS          444      *

12   ;  *     PPP        LLLLLLL   UUUUUUU    SSSSSSS           444      *

13   ;  *                                                                *

14   ;  *                                                                *

15   ;  * KKK   KKK  EEEEEEEEE  RRRRRRRR   NNN    NN     AAA     LLL     *

16   ;  * KKK  KKK   EEE        RRR   RRR  NNNN   NN   AA   AA   LLL     *

17   ;  * KKK KKK    EEE        RRR   RRR  NNNNN  NN  AAA   AAA  LLL     *

18   ;  * KKKKK      EEEEEE     RRRRRRRR   NNN NN NN  AAAAAAAAA  LLL     *

19   ;  * KKK KKK    EEE        RRR RRR    NNN  NNNN  AAA   AAA  LLL     *

20   ;  * KKK  KKK   EEE        RRR  RRR   NNN   NNN  AAA   AAA  LLL     *

21   ;  * KKK   KKK  EEEEEEEEE  RRR   RRR  NNN    NN  AAA   AAA  LLLLLLL *

22   ;  *                                                                *

23   ;  *                                                                *

24   ;  *                    V E R S I O N   3 . 5                       *

25   ;  *                                                                *

26   ;  *                                                                *

27   ;  *    COPYRIGHT (C)1984  BY   COMMODORE BUSINESS MACHINES, INC.   *

28   ;  *                                                                *

29   ;  ******************************************************************

30

31

32

33   ;  ******************************************************************

34   ;  *                                                                *

35   ;  *  THIS  SOFTWARE IS  FURNISHED FOR USE IN  COMMODORE  COMPUTER  *

36   ;  *  SYSTEMS ONLY. COPIES MAY NOT BE MADE IN WHOLE OR IN PART FOR  *

37   ;  *                   USE ON ANY OTHER SYSTEM.                     *

38   ;  *                                                                *

39   ;  *  THE  INFORMATION  IN  THIS  DOCUMENT  IS  SUBJECT  TO CHANGE  *

40   ;  *                       WITHOUT NOTICE.                          *

41   ;  *                                                                *

42   ;  *  NO  RESPONSIBILITY  IS  ASSUMED  FOR THE RELIABILITY OF THIS  *

43   ;  *                          SOFTWARE.                             *

44   ;  *                                                                *

45   ;  ******************************************************************

46

47   ;end

48

49   	.include   declare

727   ;  kennedy interface variables...

728   ;

05E7   =05E8         729   kdata	*=*+1		;temp for data write to kennedy

05E8   =05E9         730   kdycmd	*=*+1		;select for kennedy rd or wr

05E9   =05EA         731   kdynum	*=*+1		;kennedy's dev#

05EA   =05EB         732   kdyprs	*=*+1		;kennedy present::=$ff, else::=$00

05EB   =05EC         733   kdytyp	*=*+1		;temp stor. for type of open for kdy

734   ;

735   ;  and constants...

736   ;

=FEF0         737   tedrva	=$fef0

=FEF1         738   tedrvb	=$fef1

=FEF2         739   tedrvc	=$fef2

=FEF3         740   drva2	=$fef3

=FEF4         741   drvb2	=$fef4

=FEF5         742   drvc2	=$fef5

743

=0040         744   italk	=$40		;ieee talk

=0020         745   ilstn	=$20		;listen

=005F         746   iutalk	=$5f		;untalk

=003F         747   iulstn	=$3f		;unlisten

748

=0081         749   kcmd1	=$81		;state change

=0082         750   kcmd2	=$82		;sec. addr

=0083         751   kcmd3	=$83		;dout

=0084         752   kcmd4	=$84		;din

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 30

'channelio'   CHANNELIO.SRC

Error Addr  Code          Seq   Source statement

5053   ;  ***** tedisk support routines... {channel i/o} *****

5054   ;

EC8B                5055   tacptr

EC8B   86 BA        5056          stx wrbase      	;save .x

EC8D   24 F9        5057          bit usekdy      	;use kdy for rd?

EC8F   70 05        5058          bvs kdy8        	;yes b.6=1

EC91   A6 BA        5059          ldx wrbase      	;restore .x

EC93   4C E252      5060          jmp acptr       	;...else serial

5061

EC96                5062   kdy8

EC96   A5 F9        5063          lda usekdy      	;get i/o slot

EC98   29 30        5064          and #%00110000

EC9A   AA           5065          tax             	;$fec0, $fef0

EC9B   A9 84        5066          lda #kcmd4

EC9D   9D FEC0      5067          sta tedrva-48,x 	;write command

ECA0                5068   kdy85

ECA0   BD FEC2      5069          lda tedrvc-48,x 	;wait for rdy to go low

ECA3   30 FB        5070          bmi kdy85

ECA5   A9 00        5071          lda #$00

ECA7   9D FEC3      5072          sta drva2-48,x  	;setup dat/dir for inputs

ECAA   9D FEC2      5073          sta tedrvc-48,x 	;drop rdy low

ECAD                5074   kdy86

ECAD   BD FEC2      5075          lda tedrvc-48,x 	;wait for rdy to go high

ECB0   10 FB        5076          bpl kdy86

ECB2   BD FEC1      5077          lda tedrvb-48,x 	;retr status

ECB5   29 03        5078          and #3          	;mask status bits

ECB7   C9 03        5079          cmp #3          	;eoi ?

ECB9   D0 02        5080          bne kdy88

ECBB   A9 40        5081          lda #$40

ECBD                5082   kdy88

ECBD   20 F41E      5083          jsr udst        	;update status for basic

ECC0   BD FEC0      5084          lda tedrva-48,x 	;get data

ECC3   48           5085          pha             	;*

ECC4   A9 40        5086          lda #$40

ECC6   9D FEC2      5087          sta tedrvc-48,x 	;set rdy high

ECC9                5088   kdy89

ECC9   BD FEC2      5089          lda tedrvc-48,x 	;wait for dav to go low

ECCC   30 FB        5090          bmi kdy89

ECCE   A9 FF        5091          lda #$ff

ECD0   9D FEC3      5092          sta drva2-48,x  	;turn ports around i got data & status

ECD3   A9 00        5093          lda #$00

ECD5   9D FEC0      5094          sta tedrva-48,x 	;clear cmd chnl

ECD8   9D FEC2      5095          sta tedrvc-48,x 	;drop rdy low

ECDB                5096   kdy90

ECDB   4C EDD4      5097          jmp patchb      	;finish up

ECDE   EA           5098          nop             	;extra byte

5099

5100

ECDF                5101   tciout

ECDF   24 F9        5102          bit usekdy      	;use kdy for wr?

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 30-1

'channelio'   CHANNELIO.SRC

Error Addr  Code          Seq   Source statement

ECE1   30 03        5103          bmi kdy7        	;yes b.7=1

ECE3   4C E21D      5104          jmp ciout       	;else serial

5105

ECE6                5106   kdy7

ECE6   48           5107          pha             	;save (a)

ECE7   8D 05E8      5108          sta kdycmd

ECEA   A9 83        5109          lda #kcmd3

ECEC   86 BA        5110   kdy75  stx wrbase

ECEE   48           5111          pha             	;save cmd

ECEF   A5 F9        5112          lda usekdy

ECF1   29 30        5113          and #%00110000

ECF3   AA           5114          tax             	;get i/o offset

ECF4   68           5115          pla             	;retr. cmd

ECF5   9D FEC0      5116          sta tedrva-48,x 	;send cmd

ECF8                5117   kdy76

ECF8   BD FEC2      5118          lda tedrvc-48,x 	;wait for dav ack

ECFB   30 FB        5119          bmi kdy76

ECFD   AD 05E8      5120          lda kdycmd      	;ret state cmd

ED00   9D FEC0      5121          sta tedrva-48,x

ED03   A9 00        5122          lda #$00

ED05   9D FEC2      5123          sta tedrvc-48,x 	;drop dav low

ED08                5124   kdy77

ED08   BD FEC2      5125          lda tedrvc-48,x 	;wait for rdy to go high

ED0B   10 FB        5126          bpl kdy77

ED0D   BD FEC1      5127          lda tedrvb-48,x 	;retr status

ED10   29 03        5128          and #3          	;mask status

ED12                5129   kdy79

ED12   20 F41E      5130          jsr udst        	;update basic

ED15   4C EDDB      5131          jmp patcha      	;finish up

5132

5133   ;end

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 33

'openchannel'   OPENCHANL.SRC

Error Addr  Code          Seq   Source statement

5280   ;***** tedisk support routines... {state transition} *****

5281

EDA9                5282   tstkdy

EDA9   48           5283   	pha             ;save .a

EDAA   86 BA        5284          stx wrbase      	;save .x

EDAC   A2 30        5285          ldx #$30        	;starting i/o offset

EDAE   A5 AE        5286          lda fa          	;load file address

EDB0   C9 08        5287          cmp #8          	;=8?

EDB2   F0 06        5288          beq tstok

EDB4   C9 09        5289   nok8   cmp #9          	;=9?

EDB6   D0 17        5290          bne notprs

EDB8   A2 00        5291          ldx #0          	;must be $fec0

EDBA   A9 55        5292   tstok  lda #$55        	;write a pattern to cmd channel

EDBC   9D FEC0      5293          sta tedrva-48,x

EDBF   5D FEC0      5294          eor tedrva-48,x 	;is it the same

EDC2   D0 0B        5295          bne notprs

EDC4   BD FEC1      5296          lda tedrvb-48,x 	;i tied a status bit hi

EDC7   29 02        5297          and #2

EDC9   D0 04        5298          bne notprs      	;br, he is blown away

EDCB   86 F9        5299          stx usekdy      	;store offset into i/o slot

EDCD   18           5300          clc             	;ok

EDCE   24           5301          .byte $24

EDCF   38           5302   notprs sec             	;sorry not home

EDD0   A6 BA        5303          ldx wrbase      	;restore .x

EDD2   68           5304          pla             	;restore .a

EDD3   60           5305          rts             	;45 bytes

5306

EDD4   BD FEC2      5307   patchb lda tedrvc-48,x 	;wait for rdy ack to go hi

EDD7   10 FB        5308          bpl patchb

EDD9   30 05        5309          bmi ptchbb      	;bra

5310

EDDB   A9 00        5311   patcha lda #0

EDDD   9D FEC0      5312          sta tedrva-48,x 	;clear cmd channel

EDE0   A9 40        5313   ptchbb lda #$40

EDE2   9D FEC2      5314          sta tedrvc-48,x 	;set dav hi

EDE5   A6 BA        5315          ldx wrbase      	;restor .x

EDE7   68           5316          pla             	;restore .a for ciout, data for acptr

EDE8   18           5317          clc             	;ok

EDE9   60           5318          rts             	;*

5319

EDEA   8D FEF2      5320   patchd sta tedrvc

EDED   8D FEC5      5321          sta drvc2-48

EDF0   8D FEC2      5322          sta tedrvc-48

EDF3   CA           5323          dex

EDF4   8E FEC3      5324          stx drva2-48

EDF7   4C CFDD      5325          jmp ptchdd

5326

5327   ;this is in the patch area:

5328   ;ptchdd inx             ;.x=0

5329   ;       stx drvb2-48

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 33-1

'openchannel'   OPENCHANL.SRC

Error Addr  Code          Seq   Source statement

5330   ;       stx tedrva-48

5331   ;       rts             ;x must = 0 on rts !!!

5332

EDFA                5333   ttalk

EDFA   20 EDA9      5334          jsr tstkdy      	;who do we talk to???

EDFD   90 03        5335          bcc kdy1        	;he's out there

EDFF   4C E153      5336          jmp talk        	;serial dev.

5337

EE02   48           5338   kdy1   pha             	;save (a)data

EE03   A9 40        5339          lda #italk

EE05   8D 05E8      5340          sta kdycmd

EE08   A5 F9        5341          lda usekdy

EE0A   09 40        5342          ora #$40        	;write to kdy

EE0C   85 F9        5343          sta usekdy

EE0E   A9 81        5344          lda #kcmd1

EE10   4C ECEC      5345          jmp kdy75       	;finish up

5346

EE13                5347   ttkatn

EE13   24 F9        5348          bit usekdy      	;do an open in with no sa...

EE15   70 35        5349          bvs kdy5        	;kdy is pres...do nothing

EE17   4C E20C      5350          jmp tkatn       	;serial

5351

EE1A                5352   ttksa

EE1A   24 F9        5353          bit usekdy      	;do an open in with sa...

EE1C   70 03        5354          bvs kdy3        	;kdy is pres

EE1E   4C E203      5355          jmp tksa

5356

EE21                5357   kdy3

EE21   48           5358          pha             	;save (a) data

5359   ;      lda sa		; assumne SA in acc from call to TKSA by

EE22   EA           5360          NOP		; application. this is an 05 rev recommended

EE23   EA           5361          NOP		; by D. Siracusa for compatibility with LOGO, etc.

EE24   8D 05E8      5362          sta kdycmd

EE27   A9 82        5363          lda #kcmd2

EE29   4C ECEC      5364          jmp kdy75

5365

EE2C                5366   tlistn

EE2C   20 EDA9      5367          jsr tstkdy      	;do an open out with fa...

EE2F   90 03        5368          bcc kdy4        	;he's out there...

EE31   4C E156      5369          jmp listn

5370

EE34                5371   kdy4

EE34   48           5372          pha             	;save (a) data

EE35   A9 20        5373          lda #ilstn

EE37   8D 05E8      5374          sta kdycmd

EE3A   A5 F9        5375          lda usekdy

EE3C   09 80        5376          ora #$80

EE3E   85 F9        5377          sta usekdy

EE40   A9 81        5378          lda #kcmd1      	;tell tedisk to listen

EE42   4C ECEC      5379          jmp kdy75

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 33-2

'openchannel'   OPENCHANL.SRC

Error Addr  Code          Seq   Source statement

5380

EE45                5381   tscatn

EE45   24 F9        5382          bit usekdy      	;do an open out with no sa...

EE47   30 03        5383          bmi kdy5

EE49   4C E1FC      5384          jmp scatn

5385

EE4C                5386   kdy5

EE4C   60           5387          rts             	;do nothing for kdy

5388

EE4D                5389   tsecnd

EE4D   24 F9        5390          bit usekdy      	;do an open out with sa...

EE4F   30 03        5391          bmi kdy6

EE51   4C E1F7      5392          jmp secnd

5393

EE54                5394   kdy6

EE54   48           5395          pha             	;save (a) data

EE55   8D 05E8      5396          sta kdycmd

EE58   A9 82        5397          lda #kcmd2

EE5A   4C ECEC      5398          jmp kdy75

5399

5400   ;end

ted_kernal	CR6502/11 version 20.51.10	14-Aug-84  17:47:41	Page 35

'close all files'   CLALL.SRC

Error Addr  Code          Seq   Source statement

5598   ;***** tedisk support routines {state change} *****

5599   ;

EF23                5600   tunlsn

EF23   24 F9        5601          bit usekdy      	;unlisten patch

EF25   30 03        5602          bmi kdy9        	;kdy was listening...

EF27   4C E23D      5603          jmp unlsn       	;serial was...

5604

EF2A                5605   kdy9

EF2A   48           5606          pha             	;save (a)

EF2B   A9 3F        5607          lda #iulstn

EF2D   8D 05E8      5608          sta kdycmd

EF30   A5 F9        5609          lda usekdy      	;status shows unlisten

EF32   29 7F        5610          and #$7f

EF34   85 F9        5611          sta usekdy

EF36   A9 81        5612          lda #kcmd1

EF38   4C ECEC      5613          jmp kdy75       	;send cmd

5614

EF3B                5615   tuntlk

EF3B   24 F9        5616          bit usekdy      	;untalk patch

EF3D   70 03        5617          bvs kdya

EF3F   4C E22F      5618          jmp untlk

5619

EF42                5620   kdya

EF42   48           5621          pha             	;save (a)

EF43   A9 5F        5622          lda #iutalk

EF45   8D 05E8      5623          sta kdycmd

EF48   A5 F9        5624          lda usekdy

EF4A   29 BF        5625          and #$bf

EF4C   85 F9        5626          sta usekdy

EF4E   A9 81        5627          lda #kcmd1

EF50   4C ECEC      5628          jmp kdy75       	;send cmd

5629

5630   ;end